# TECHNICAL SPECIFICATION© ISO 2001 – All rights reservedISO/TS 13499:2001(E) 27 Road vehicles — Multimedia data exchange format for impact testsVéhicules routiers — Format d'échange de données multimédia pour les essais de chocRoad vehicles — Multimedia data exchange format for impact testsE2001-10-11(60) PublicationISOISO    Technical Specification ISO/TS 13499ISO/TS 13499ISO 13499:2001(E) AFNOR Road vehicles31222 2Titre 2Titre 1    Fr1  Preview release 3STD Version 2.060   4\SERV_CESPA\02\_BNA\XDOM001\ISO\ISO\ISO_TS13499_(E).doc             ISO TC 22/SC 12

Date:   2024-07-xx

# ISO/TS 13499 – RED A : 2024(E)

# ISO TC 22/SC 36/WG 3

# Secretariat:   AFNOR

# Road vehicles — Multimedia data exchange format for impact tests

# Véhicules routiers — Format d'échange de données multimédia pour les essais de choc

# Related electronic document A

# Examples and Hints

- Version 1.6  —

Revision 3

# Contents		Page

A.1	Examples of files	2

A.1.1	Example of test descriptor file	2

A.1.2	Example of test comment file	3

A.1.3	Example of channel comment file	3

A.1.4	Example of diagram comment file	3

A.1.5	Example of movie comment file	3

A.1.6	Example of photo comment file	3

A.1.7	Example of report comment file	3

A.1.8	Example of static comment file	4

A.1.9	Example of channel information file	4

A.1.10	Example of test channel file	4

A.1.11	Examples of moving image information files	5

A.1.11.1	Example of moving image information file	5

A.1.11.2	Example of image history file	6

A.1.11.3	Example of correction parameter file for bundle adjustment	6

A.1.12	Example of photo information file	6

A.1.13	Example of static data file	7

A.2	Hints and changes	8

A.2.1	MME file	8

A.2.1.1	MME file - Additional descriptors	8

A.2.1.2	Barrier information descriptors	8

A.2.1.3	 Time of the test information	8

A.2.2	Test channel files, CHN file and channel comment file	9

A.2.2.1	Test channel files - Additional descriptors	9

A.2.2.2	Test channel files and CHN file - Hint: Sign convention	10

A.2.2.3	Test channel files - Hint: Order of fine locations	10

A.2.2.4	Test channel files - Hint: Data from film analysis	10

A.2.2.5	Test channel files - Hint: Barrier load cell channel code	10

A.2.2.6	Channel comment file - Hint: Channel specific information	10

A.2.2.7	Channel file – Additional descriptors for load cells	10

A.2.2.8	Test channel files - Hint: Channel numbering	11

A.2.2.9	Test channel files – Transducer damping ratio	11

A.2.2.10	Test channel files – Transfer function	11

A.2.3	MII file	12

A.2.3.1	MII file - Correction	12

A.2.3.2	MII file - Hint: Single image sequences	12

A.2.3.3	MII file - Additional descriptors for 3D film analysis	13

A.2.3.4	MII file -  Hint for AVI format files	13

A.2.3.5	MII file - Additional information about the history of image processing and distortion correction	13

A.2.3.5.1	MII file - Additional descriptors for image processing and distortion correction	13

A.2.3.5.2	Structure of an image history file	14

A.2.3.5.3	Structure of a correction parameter file for bundle adjustment	14

A.2.4	PHO file	14

A.2.4.1	PHO file - Correction	14

A.2.5	Data exchange with the NHTSA	15

A.2.6	Calculated value codes and channels	15

A.2.7	Channel code – Hint: Naming convention for the position field (Motorcycle)	15

A.2.8	MME directory structure – Hint: Missing media objects	15

A.2.9	Optional descriptors (Dot-Descriptors)	15

A.2.10	Line length of information files	15

A.2.11	Case sensitivity of filenames	15

A.2.12	Combination of dummy parts	15

A.2.13	MME file - Test object information	15

A.2.14	Naming rules for channels	16

A.2.15	MME directory structure – Recommendations for special data	16

# RED A : Examples and Hints

# A.1	Examples of files

## A.1.1	Example of test descriptor file

File name :       2005ISO1.MME

Data format edition number	:1.6

Laboratory name	:ALPHA Car Test Laboratory

Laboratory contact name	:Frank N. Stein

Laboratory contact phone	:+49-222/123-4567

Laboratory contact fax	:+49-222/123-8901

Laboratory contact email	:frank.stein@alpha.cartest.com

Laboratory test ref. number	:2005WG3

Customer name	:ISO/TC22/SC12/WG3 Safety Laboratory

Customer test ref. number	:2005ISO1

Customer project ref. number	:ISOTC22

Customer order number	:SC12WG3

Customer cost unit	:2001/0

Customer test engineer name	:Mary Land

Customer test engineer phone	:+44-123/555-123

Customer test engineer fax	:+44-123/555-456

Customer test engineer email	:mary.land@iso.tc22.sc12.wg3.uk

Title	:Simulation Test

Medium No./number of media	:1/1

Timestamp	:2005-05-22 14:25:15

Type of the test	:Frontal

Subtype of the test	:40% Offset left 0°

Regulation	:EuroNCAP

Reference temperature	:285.5

Relative air humidity	:75

Date of the test	:2005-05-22

Number of test objects	:2

Comments	:

Comments	:  The following block describes test object 1

Comments	:

Name of test object 1	:Vehicle A

Velocity test object 1	:15.72

Mass test object 1	:1430.00

Driver position object 1	:1

Impact side test object 1	:FR

Type of test object 1	:1

Class of test object 1	:Small Family Car

Code of test object 1	:A05

Ref. number of test object 1	:007-008

.Offset 1	:40

Comments	:

Comments	:  The following block describes test object 2

Comments	:

Name of test object 2	:Fixed barrier with load cell matrix

Velocity test object 2	:0.00

Mass test object 2	:NOVALUE

Driver position object 2	:NOVALUE

Impact side test object 2	:FR

Type of test object 2	:B

Class of test object 2	:NOVALUE

Code of test object 2	:NOVALUE

Ref. number of test object 2	:NOVALUE

.Barrier width 2	:3.2

.Barrier height 2	:1.64

.Yaw angle 2	:-1.507

.Reference system 2	:laboratory

Comments	 according to the SAE J211 coordinate system

.Origin X 2	:0.12

.Origin Y 2	:-1.4

.Origin Z 2	:-1.8

.Number of loadcells 2	:64

## A.1.2	Example of test comment file

# File name :       2005ISO1.TXT

The car to car test was performed on 22nd of May 2005 at ALPHA Car Test Laboratory.	
The airbags of vehicle A had to be exchanged before the test. ...

## A.1.3	Example of channel comment file

File name :       CHANNEL.TXT

Following problems occurred:
11HEAD0000H3ACXA: large deviations at post test calibration ...

## A.1.4	Example of diagram comment file

# File name :       DIAGRAM.TXT

# The plots and diagrams of the test are stored as postscript files. ...

## A.1.5	Example of movie comment file

File name :       MOVIE.TXT

The 16 mm films are scanned with an EPSILON film scanner in 1.020 x 1.360 pixel resolution. The images are resized to 576 x 768 pixel with the program TAU. The creation of the AVI container files was performed with the program AVIMAKER. ...

## A.1.6	Example of photo comment file

# File name :       PHOTO.TXT

All pictures photographed with the ETA 007 are resized from 5.850 x 5.000 to 1.170 x 1.000 pixel. The resizing and the colour correction are performed with the program TAU. ...

## A.1.7	Example of report comment file

File name :       REPORT.TXT

The name of the report file is 2005ISO1_REPORT.TXT. It contains plain text with up to 80 characters per line and each line is separated by a ‚carriage return‘ and a ‚line feed‘ (CR/LF). ...

## A.1.8	Example of static comment file

# File name :       STATIC.TXT

The ID-number of the points starts with 7, because the first 6 points define our reference system for frontal impacts. ...

## A.1.9	Example of channel information file

File name :       2005ISO1.CHN

Instrumentation standard	:ISO 6487 (1987)  /  SAE J211 (MAR95)

Number of channels	:13

Name of channel 001	:11HEAD0000H3ACXA	/ Head Acceleration X

Name of channel 002	:11HEAD0000H3ACYA	/ Head Acceleration Y

Name of channel 003	:11HEAD0000H3ACZA	/ Head Acceleration Z

Name of channel 004	:11CHST0000H3DSXC	/ Chest Displacement X

Name of channel 005	:14BPILLO0000ACXC	/ B-Pillar Acceleration X

Name of channel 006	:11NECKUP00H3FOZA	/ Neck upper Force Z

Name of channel 007	:11NECKUP00H3MOYB	/ Neck upper Moment Y

Name of channel 008	:B0LOMA007525FOXP	/ Loadcell matrix Force X

Name of channel 009	:B0LOMA007550FOXP	/ Loadcell matrix Force X 
Name of channel 010	:B0LOMA007575FOXP	/ Loadcell matrix Force X

Name of channel 011	:11TIBILEUPH3FOZB	/ Tibia left upper Force Z

Name of channel 012	:11TIBILEUPH3MOXB	/ Tibia left upper Moment X

Name of channel 013	:11TIBILEUPH3MOYB	/ Tibia left upper Moment Y

## A.1.10	Example of test channel file

# Filename :       2005ISO1.001

Test object number	:1

Name of the channel	:Head Acceleration X

Laboratory channel code	:HEAD01AX

Customer channel code	:1HEAD_X_ACC

Channel code	:11HEAD0000H3ACXA

Comments	: next 4 items are optional and part of the channel code

Location	:Head		(=HEAD	at code position 3-6)

Dimension	:Acceleration	(=AC	at code position 13-14)

Direction	:Longitudinal	(=X	at code position 15)

Channel frequency class	:1000		(=A	at code position 16)

Unit	:m/(s*s)

Reference system	:Local

Transducer type	:TAU 7270 A

Transducer id	:42

Pre-filter type	:Butterworth, 6 pole

Cut off frequency	:2000.0

Channel amplitude class	:2000.0

Reference channel	:implicit

Reference channel name	:NOVALUE

Data source	:transducer

Data status	:ok

Sampling interval	:0.0001

Bit resolution	:12

Time of first sample	:0.0000

Number of samples	:2500

Offset post test	:0.15

Transducer natural frequency	:27000

Transducer damping ratio	:0.5

Comments	: next 6 items for proofing

First global maximum value	:+1.237802E+02

Time of maximum value	:+0.18450

First global minimum value	:-5.489905E+02

Time of minimum value	:+0.06860

Start offset interval	:-0.0500

End offset interval	:+0.0000

-4.788391E-01

-7.182586E-01

. . .
+3.064578E+01

## A.1.11	Examples of moving image information files

### A.1.11.1	Example of moving image information file

# File name :       2005ISO1.MII

Number of movies	:7

Comments	:

Comments	: the following block describes movie 1

Comments	:

ID-number 1	:1

Origin 1	:Crashtest

Description 1	:total view of vehicle A from the left side

Camera type 1	:KAPPA ROC

Camera ID-number 1	:KAPPA12

Lens ID-number 1	:14579435

Lens type 1	:Schneider

Focus 1	:infinite

Lens focal length 1	:10

Number of images 1	:350

Film speed 1	:1000

Shutter time 1	:0.0001

Aperture 1	:5.6 - 8

Time zero 1	:50

Time vector filename 1	:NO

Reference system 1	:middle of the test block, on the floor

Location X 1	:-25.0

Location Y 1	:-9.5

Location Z 1	:1.2

Theta X 1	:90

Theta Y 1	:0

Theta Z 1	:270

Width of image 1	:512

Height of image 1	:384

Aspect ratio of pixels 1	:1.00

Colour 1	:RGB

Name of movie file 1	:LEFTATOT.AVI

Format of movie file 1	:AVI

Keyframes 1	: 3

Codec used 1	:Indeo video 5.11 (change to modern codec?)

Compression 1	:5.1 %

Distortion index 1	:NOVALUE

Movie images corrected 1	:NO

Correction parameter file 1	:KAPPA12_14579435.COR

Image history filename 1	:KAPPA12.IMH

Comments	:

Comments	: the following block describes movie 2

ID-number 2	:2

. . .

### A.1.11.2	Example of image history file

File name :       see MII file descriptor   Image history filename i

Image processing system	:SICON V5.2.3

Image interpolation	:SICON V1.2

Sharpening 1	:5

Colour matrix 1	:R(113 / 38 / -50), G(-25 / 144 / -19),  B(-25 / -50 / 175)

White balance 1	:175 / 175 / 240

Brightness 1	:R(15 / 135 / 255), G(15 / 135 / 255), B(15 / 135 / 255)

Contrast 1	:NOVALUE

Saturation 1	:1.0

Hue 1	:NOVALUE

Gamma 1	:1.5

Sharpening 2	:8

### A.1.11.3	Example of correction parameter file for bundle adjustment

File name :       see MII file descriptor   Correction parameter file i

Distortion correction type	:bundle adjustment

Pixel distance x	:0.016

Pixel distance y	:0.016

Principal point x	:-9.38

Principal point y	:-8.25

Calibrated focal length	:10.128

Distortion unit	:mm

Distortion correction A1	:-1.1685e-003

Distortion correction A2	:5.3873e-006

Distortion correction A3	:2.8685e-007

Distortion correction B1	:-1.4558e-005

Distortion correction B2	:-3.2337e-005

Distortion correction C1	:6.6139e-007

Distortion correction C2	:3.6798e-005

Distortion correction R0	:3.413

+Method	:simple

+Date of the calibration	:2005-03-03

## A.1.12	Example of photo information file

File name :       2005ISO1.PHO

Number of photos	:6

Comments	:

Comments	: the following block describes photo 1

Comments	:

ID-number 1	:1

Test object number	:2

Camera type 1	:ETA 007

Post-test / Pre-test 1	:POST

Description 1	:partial view of the front end of vehicle B

Direction 1	:right

Aperture 1	:8.0

Exposure time 1	:0.008

Comments	: original area of the camera chip 5850 x 5000 pixel

Width of image 1	:1170

Height of image 1	:1000

Aspect ratio of pixels 1	:1.00

Colour 1	:RGB

Name of photo file 1	:BRIGPOST.TIF

Format of photo file 1	:TIFF

Compression 1	:LZW

Comments	:

Comments	: the following block describes photo 2

ID-number 2	:2

. . .

## A.1.13	Example of static data file

File name :       2005ISO1.SD1

Number of measurements	:30

Comments	: the following block describes point 1

Comments	: the first point of the origin

ID-number 1	:7

. . .

Comments	: the following block describes point 7

ID-number 007	:13

Description 007	: Suspension dome right

X Pre 007	:0.017

Y Pre 007	:-0.466

Z Pre 007	:0.545

X Post 007	:0.287

Y Post 007	:-0.398

Z Post 007	:0.589

Comments	: the following block describes point 8

ID-number 008	:14

Description 008	: Sill at B-Pillar

. . .

# A.2	Hints and changes

## A.2.1	MME file

### A.2.1.1	MME file - Additional descriptors

To handle the information about the test type more automatically it is meaningful to introduce additional descriptors to specify the subtype of a certain main test type and the regulation for the test. If for example the test type is a "frontal impact" possible values for subtype and regulation could be “ODB” and “Euro NCAP”.
 For every test object (m) the offset value in percent can be described by an optional descriptor.

Additional descriptors for the specification of a test:

Subtype of the test	alphanumeric

Regulation	alphanumeric	Example in the style of V2.0 variants?

.Offset m	float	Overlap in percent

Additional descriptors related to the test object velocity:

Nominal vel. test object x	float	Intended velocity for TOB no “x”

Lat. vel. test object x     	float	lateral velocity for TOB no “x”

Nom. lat. vel. test object x	float	intended lateral velocity for TOB no “x”

### A.2.1.2	Barrier information descriptors

If the test object m is a barrier, optional descriptors can be used to describe its orientation and loadcell matrix. The coordinate system according to SAE J211/1 MAR95 Instrumentation for Impact Test is described in A.2.2.5

.Barrier width m	float	[m]

.Barrier height m	float	[m]

.Yaw angle m	float	Angle of barrier with normal to direction of vehicle travel. Units: radians. limited to /2. 0 rad means that the barrier is perpendicular to the vehicle. Positive sense: Clockwise when viewed from above (SAE J211)

.Reference system m	alphanumeric	Coordinate reference system e.g. laboratory

.Origin X m	float	[m] Top left corner of the loadcell matrix within the reference system - X coordinate

.Origin Y m	float	[m] Top left corner of the loadcell matrix within the reference system - Y coordinate

.Origin Z m	float	[m] Top left corner of the loadcell matrix within the reference system - Z coordinate

.Number of loadcells m	integer

### A.2.1.3		Time of the test information

In some applications the time of the test is needed to identify the sequence of multiple tests. The time information (as local time without time zone) can be added to the ‘Date of the test’ descriptor. The format should comply to the ‘Timestamp’ descriptor e.g.:

Date of the test            :2024-07-08 11:19:04

## A.2.2	Test channel files, CHN file and channel comment file

### A.2.2.1	Test channel files - Additional descriptors

Additional descriptors for time channel reference:

Reference channel	possible values:

implicit	Time reference is given with the descriptor values
		‘Time of first sample‘ and ‘Sampling interval‘

explicit	Explicit time channel exists in test data. Channel name
		is given with the descriptor ‘Reference channel name‘

NOVALUE	No time reference is available. For example in case of
		constant channels (filter class ‘X‘).

Reference channel name	Name of reference channel in test data if reference channel value
	‘explicit‘ is required. Then use ??TIRS??????TI?0 otherwise NOVALUE

Additional descriptor for channel data source and status:

Data source	possible values:

transducer 	Channel data has been generated by transducer

calculation 	Channel data has been calculated from other channels

camera	Channel data has been generated by film analysis

simulation	Channel data has been generated by simulation

parameter	Channel data can be constant or limit curve

Data status	possible values: see table below

The ‘Data Status’ property has been reviewed and redefined in (2023-2024) to allow a distinct decision of the validity of the data for the analysis/processing. The new categories are:

| Data Status | Explanation | Data modified | Valid for Analysis |
| --- | --- | --- | --- |
| ok | faultless sample data | no | yes |
| no data | no sample data available or NOVALUEs only | no | no |
| channel failed | channel failed completely or in the important time range, if caused e.g., by the sensor or sensor cable | no | no |
| system failed | channel failed completely or in the important time range, if caused e.g., by the data recorder | no | no |
| questionable data | needs to be validated | no | no |
| partially failed | failure is outside the important time range | no | yes |
| corrected data | data have been corrected (e.g., by multiplying by a certain factor ("scaling factor applied", also inverting) or by interpolating parts of the channel ("linearised data") so that they can be used for assessment; excluding: renaming, offset correction ! | yes | yes |
| NOVALUE | no status information available (e.g., in case of raw data) | ? | no |

Tab 1: Data Status values and applicability

The former categories for the ‘Data status’ can be translated into the new classification:

| Old Data Status | New Data Status |
| --- | --- |
| ok | ok |
| channel failed | channel failed |
| meaningless data | use "partially failed" |
| no data | no data |
| questionable data | questionable data |
| scaling factor applied | use "corrected data" |
| system failed | system failed |
| linearised data | use "corrected data" |
| NOVALUE | NOVALUE |

Tab 2: Transfer/Translate old ‘Data Status’ categories

The descriptor “Errors occurred” should not be used any longer.

Additional descriptor for transducers:

Transducer id	alphanumeric

The optional descriptor for the dimension of a channel is missing in the main document:

Dimension	alphanumeric	see ‘Dimension’ in related electronic 
document "Channel codes"

### A.2.2.2	Test channel files and CHN file - Hint: Sign convention

The directions X, Y and Z of RED B refer to the SAE J211 sign convention. For any other sign convention, which shall be defined as value of the descriptor ‘Instrumentation standard’ in the Channel Information File or as value of the descriptor ‘Reference system’ in every Test Channel File, the directions 1, 2 and 3 shall be used.

### A.2.2.3	Test channel files - Hint: Order of fine locations

If there is a need for more than one fine location in the channel code, you shall use LE/RI for fine location 1.

### A.2.2.4	Test channel files - Hint: Data from film analysis

For the exchange of trajectories from film analysis it is possible to use the channel files. Up to now no specific channel code for such data is defined, but the code described in the RED B may be used. Therefore, the trajectories shall be split into the distinct directions. Test Object, Position, Main Location, Fine Locations and Direction are also usable similar to transducer channels. To distinguish between film analysis data and other data use “V” as Filter Class for data from film analysis. For the descriptor ‘Data source’ you shall use the value camera.

### A.2.2.5	Test channel files - Hint: Barrier load cell channel code

For barrier load cells the channel code has to be M?LOMA??nnmm???? for mobile and B?LOMA??nnmm???? for fixed barriers, where nn gives the row in percent of the total extend in Z-direction and mm gives the column in percent of the total extend in Y-direction of the load cell matrix. The values nn and mm are the positions of the load cell center rounded to the nearest integer. The barrier is seen as a vehicle with a coordinate system according to SAE J211/1 MAR95 Instrumentation for Impact Test. This means the row points into the Y-, the column into the Z-direction and The origin is in the left upper corner of the load cell matrix looking from the barrier to the other test object.

See Figure in RED C….

### A.2.2.6	Channel comment file - Hint: Channel specific information

Channel codes may also be used to mark channel specific information.

Example line?

### A.2.2.7	Channel file – Additional descriptors for load cells

This file contains information concerning the specific barrier loadcell channel and all its measurement values, expressed in physical units and balanced. Its structure complies to a test channel file with the following additional descriptors.

.Width	float	[mm]  width of loadcell
.Height	float	[mm]  height of loadcell
.Top left Y	float	[mm]  defines top, left of loadcell 
		wrt the loadcell matrix origin
.Top left Z	float	[mm] defines top, left  of loadcell 
		wrt the loadcell matrix origin

### A.2.2.8	Test channel files - Hint: Channel numbering

If the channel number is greater than 999 the number of digits of the channel number “NNN” can be expanded. Channels with a number up to 999 will keep their 3-digit file extension. Channels with a higher number will use as much digits as needed:

<testname>.001
<testname>.002
...
<testname>.999
<testname>.1000
<testname>.1001
...

### A.2.2.9	Test channel files – Transducer damping ratio

For the description of the overall frequency response of a transducer channel and the entire data collection system two new mandatory descriptors are necessary.

Transducer natural frequency	float	[Hz]

Transducer damping ratio	float	[1]

### A.2.2.10	Test channel files – Transfer function

For a long time, only linear regression has been used to describe the calculation of physical units from the transducer signal. New types of transducers and new regulations require an extended concept of the describing parameters. The way from the transducer signals to the exchanged data can be described as a set of transfer functions.

| Use case | Application | Transfer function |
| --- | --- | --- |
| Linear transducer with relative measurement | Standard for most transducers | Linear regression |
| Linear transducer with absolute measurement | Absolute pressure, angle transducer in MTRACs | Linear regression with offset |
| Nonlinear transducer with polynomial approximation | Polynomial calibrated belt force | Polynomial re-gression 3rd order |
| Nonlinear transducer with power function approximation | Displacement transducer in the IR-TRACC | Power function |
| Linear transducer with relative measurement and additional polynomial calibration1) | Chest deflection of H III Dummy | Linear regression |

Tab 3:	Use cases for transfer functions

1) In the case of chest deflection measurement, the data has to be calculated and exchanged with the Linear regression transfer function. In the channel file header, the used parameters and in addition the parameters necessary for the calculation of the Polynomial regression 3rd order transfer function have to be stored. This functionality needs different descriptors for both transfer functions.

For all use cases described in Tab 1 a specific set of descriptors are defined in Tab 2. The descriptors of the transfer functions are in principle optional, but if a specific function is named in the channel header, the set of associated descriptors has to be complete.

| Transfer function | Formula |  | Descriptor | Unit |
| --- | --- | --- | --- | --- |
| Necessary for all functions | D : physical value [Unit] S : sensor output reading [eU] |  | .Transfer function used .Direction polarity Offset post test | - - [Unit] |
| Linear regression | D = C * S | C | Inverse sensitivity Offset pre test .Sensitivity2) | [Unit/eU] [eU] [eU/Unit] |
| Linear regression with offset | D = C * S + M | C M | Inverse polynom coeff C Inverse polynom coeff M Offset pre test | [Unit/eU] [Unit] [eU] |
| Polynomial regression 3rd order | D = A * S3 + B * S2 + C * S + M | A B C M | Inverse polynom coeff A Inverse polynom coeff B Inverse polynom coeff C Inverse polynom coeff M | [Unit/(eU)3] [Unit/(eU)2] [Unit/eU] [Unit] |
| Power function | D = C * (S + S0)α + M | C M S0 α | .Power func sensitivity .Power func eng offset .Power func electr offset .Power func exponent | [Unit/eU] [Unit] [eU] [1] |

Tab 4:	Parameters and descriptors of transfer functions

2) Additional information: must not be used if ‘Inverse sensitivity’ is available and not NOVALUE.

The function Linear regression with offset is a special case of Polynomial regression with offset where the cubic and quadratic coefficients are 0.

Independent from the transfer function the descriptor ‘Offset post test’ can be used to document the offset correction in physical units in the time between ‘Start offset interval’ and ‘End offset interval’.

## A.2.3	MII file

### A.2.3.1	MII file - Correction

The data format for the descriptor 'Aperture i' was specified as a float value. In practice it is often necessary to store an intermediate value like “5.6 – 8”. Therefore, the new data format specification is alphanumeric.

### A.2.3.2	MII file - Hint: Single image sequences

In most cases of movie exchange container files like AVI are used. Especially for film analysis it could be necessary to exchange the original single images. To avoid equivocal filenames the best method is to store every sequence of single images within an own subdirectory of the movie directory. The name of the subdirectory shall be equal to the container filename without extension. The names of the single image files shall contain numbers for sorting.
The descriptive parameters of the container file and the image sequence are partly different. Therefore, it is necessary to use two distinct information blocks within the MII file if both are exchanged. For the whole image sequence only one descriptor 'Name of moviefile i' can be used. It shall contain only the subdirectory name without dots and pathnames.

Example for the image sequence F00000.IMG ... F00299.IMG in the subdirectory LEFT1 together with the container file LEFT1.AVI:

...

Number of images 1	200

Time zero 1	5

Width of image 1	512

Height of image 1	512

Colour 1	RGB

Name of movie file 1	LEFT1.AVI

Format of movie file 1	AVI

Codec used 1	Indeo video 5.11 (change to modern codec?)

Compression 1	5 %

...

Number of images 2	300

Time zero 2	10

Width of image 2	1024

Height of image 2	1024

Colour 2	RGB

Name of movie file 2	LEFT1

Format of movie file 2	TIFF

Codec used 2	uncompressed

Compression 2	NOVALUE

...

# All other descriptive data should be equal for both kinds of movie.

### A.2.3.3	MII file - Additional descriptors for 3D film analysis

Camera ID-number i	alphanumeric

Lens ID-number i	alphanumeric

Lens type i	alphanumeric

Focus i	alphanumeric

### A.2.3.4	MII file -  Hint for AVI format files

The recommended compression algorithm for the use in AVI files is X264 Revision 1195. This codec may be used with the following parameters:

| Single pass – ratefactor-based (CRF) |  |
| --- | --- |
| Ratefactor | 23 or less |
| FourCC | X264 |
| ME algorithm | umh |
| Subpixel ME refinement | 7 RD on all |
| Min. GOP Size | 7 |
| Max. GOP Size | 7 |
| Max. Number of consecutive B-Frames | 0 |

The first frame of an AVI file has the number 0. It has to be considered for the calculation of the “Time zero” value.

### A.2.3.5	MII file - Additional information about the history of image processing and distortion correction

#### A.2.3.5.1	MII file - Additional descriptors for image processing and distortion correction

The history of the image processing and also the parameters for the distortion correction may be stored in special files in the movie subdirectory referenced by:

Image history filename i	alphanumeric	according to the filename convention

Correction parameter file i	alphanumeric	according to the filename convention

Additional information about the distortion correction and the quality of the optical chain shall be given by:

Distortion index i	float	[%] Distortion index according to ISO 8721

Movie images corrected i	alphanumeric	YES or NO

are the images corrected with the

distortion parameters?

#### A.2.3.5.2	Structure of an image history file

The image history file is optional. The descriptors are not mandatory. They are unique but their position order shall show the time history of the single processing steps. The numbering of the descriptors is used to decide between twice or more usage of the same processing item.

Image processing system	alphanumeric	with version number

Image interpolation	alphanumeric	with version number

Sharpening i	alphanumeric	typically 1 integer value

Colour matrix i	alphanumeric

White balance i	alphanumeric	typically 3 integer values

Brightness i	alphanumeric	single or matrix of integer

Contrast i	alphanumeric 	single or matrix of integer

Saturation i	alphanumeric 	integer or float

Hue i	alphanumeric 	typically integer

Gamma i	alphanumeric	typically 1 float value

#### A.2.3.5.3	Structure of a correction parameter file for bundle adjustment

The correction parameter file is optional. The file content for the correction method bundle adjustment shall be:

Distortion correction type	alphanumeric	bundle adjustment

Pixel distance x	float	[mm]

Pixel distance y	float	[mm]

Principal point x	float	[pixel] deviation from the centre of the image

(positive from left to right)

Principal point y	float	[pixel] deviation from the centre of the image

(positive from top to bottom)

Calibrated focal length	float	[mm]  as positive value

Distortion unit	alphanumeric	pixel or mm 
		unit for all following distortion parameters

Distortion correction A1	float	1. corr. coeff. for radial symmetrical distortion

Distortion correction A2	float	2. corr. coeff. for radial symmetrical distortion

Distortion correction A3	float	3. corr. coeff. for radial symmetrical distortion

Distortion correction B1	float	1. corr. coeff. for radial asymmetrical distortion

Distortion correction B2	float	2. corr. coeff. for radial asymmetrical distortion

Distortion correction C1	float	affinity

Distortion correction C2	float	non-orthogonality

Distortion correction R0	float	2. zero crossing of the distortion curve

# Additional descriptors shall start with a ‘+’ sign. (Do we have this in a prominent general statement? See A.2.9)

# The descriptors for other correction methods shall be agreed between the partners.

## A.2.4	PHO file

### A.2.4.1	PHO file - Correction

The data format for the descriptor 'Aperture i'  was specified as a float value. In practice it is often necessary to store an intermediate value like “5.6 – 8”. Therefore, the new data format specification is alphanumeric.
The allowed values for the descriptor ‘Post-test / Pre-test i’ are changed to POST, PRE, DURING, NOVALUE.

## A.2.5	Data exchange with the NHTSA

The additional information for data exchange with the NHTSA shall be placed in special files in the subdirectory NHTSA. For detailed specification see RED D Version 1.6 .

## A.2.6	Calculated value codes and channels

It is possible now to exchange calculated value codes and channels. For detailed specification see RED E Version 1.6 .

## A.2.7	Channel code – Hint: Naming convention for the position field (Motorcycle)

For special seat arrangements with different rows or more than three seats per row the usage of capital letters A - Z for the position field is recommended. Therefore, the old code value “D” for Motorcycle Driver is now changed into “2”, the old code value “S” for Motorcycle Sozius/Pillon rider is changed into “5”.

## A.2.8	MME directory structure – Hint: Missing media objects

Subdirectories and their information files for missing media objects are no longer mandatory.

## A.2.9	Optional descriptors (Dot-Descriptors)

Optional descriptors can be introduced in all ISO-MME information files. They have to begin with a “.”.

## A.2.10	Line length of information files

The lines of all information files can exceed 80 characters if agreed between the transferring parties.

## A.2.11	Case sensitivity of filenames

Parts of filenames and extensions like the testnumber, which occur several times in the file structure, have to be identical in the usage of upper- and lower-case letters.

## A.2.12	Combination of dummy parts

When combining parts of different dummy types, a unique FineLocation3 has to be used. Esp. when using THOR legs together with the 50th percentile male Hybrid III the FineLocation3 should be T3 for all dummy channels.

## A.2.13	MME file - Test object information

For the descriptor ‘Class of test object m’ the EuroNCAP class or the platform type is recommended for vehicles. The descriptor ‘Code of test object m’ should refer to the code of model type or stage of development for vehicles and to the code of Fine Location 3 for pedestrian impactors. The ‘Ref. number of test object m’ should be a unique identification number like the vehicle Id.

## A.2.14	Naming rules for channels

For the Channel Codes the following general naming rules apply:

• The MainLocation should describe the position of the measurement point.

• ChannelCodes which deviate from the following rules but are in use for a long time should not be changed.

Especially for new dummy and impactor channels the following naming rules apply:

• The naming of the channels should describe the structure of the dummy as a measurement device.

• The FineLocation1 should describe the most significant spatial orientation of the channel.

• The FineLocation2 should describe the less significant characteristic of the channel location or property.

• The FineLocation3 is identical to the dummy code.

In addition, for all other channels where the FineLocation3 is not reserved for the dummy code the following rules apply:

• The FineLocation 1,2,3 should be used in the order X,Y,Z in the coordinate system of the testobject, if all three spatial directions are used.

• If one or two spatial directions are unused the other move up, resulting in 00 values at the FineLocations 3 and possibly 2.

## A.2.15	MME directory structure – Recommendations for special data

For the exchange of data files with CAN-Bus or FlexRay protocols the subdirectory Busdata should be used.

For the exchange of simulation input- or output-decks the subdirectory Simulation should be used.

The additional data according ISO TS 23520 ‘Equipment eXchange – data format specification for operational information relevant for equipment exchange and testing’ should be exchanged in the subdirectory Equipment.