TECHNICAL SPECIFICATION© ISO 2001 – All rights reservedISO/TS 13499:2001(E) 27 Road vehicles — Multimedia data exchange format for impact testsVéhicules routiers — Format d'échange de données multimédia pour les essais de chocRoad vehicles — Multimedia data exchange format for impact testsE2001-10-11(60) PublicationISOISO    Technical Specification ISO/TS 13499ISO/TS 13499ISO 13499:2001(E) AFNOR Road vehicles31222 2Titre 2Titre 1    Fr1  Preview release 3STD Version 2.060   4\SERV_CESPA\02\_BNA\XDOM001\ISO\ISO\ISO_TS13499_(E).doc             ISO TC 22/SC 36

Date:   2026-07-11

ISO/TS 13499 – RED F : 2026(E)

ISO TC 22/SC 36/WG 3

Secretariat:   AFNOR

Road vehicles — Multimedia data exchange format for impact tests

Véhicules routiers — Format d'échange de données multimédia pour les essais de choc

Related electronic document F

Active Safety Recommendation

Rev 20260711

# Contents		Page

1	Normative references	4

2	Hints	4

3	MME Descriptors	4

3.1	Remarks to MME descriptors for active safety	4

3.2	Additional MME descriptors for active safety	4

3.3	MME Attribute Values	7

3.3.1	Test object	7

3.3.2	System Function	7

3.3.3	Scenario	8

3.3.4	Test Campaign Type	9

3.3.5	Type of the test	10

3.3.6	Subtype of the test	10

3.3.7	Regulation	11

3.3.8	Name of test object X	11

3.3.9	Vehicle Shape and Vehicle Dimension	12

3.3.10	Impact locations, heading and side definitions	12

3.3.11	Robustness Layer	13

4	Reference Coordinate Systems	15

4.1	Vehicle Coordinate System	16

4.2	Test Ground Coordinate System	16

5	Channel Codes	17

5.1	Enhancement for specific Main and Fine Locations for active safety	17

5.1.1	Additional Main Locations	17

5.1.1.1	Function based Main Locations	17

5.1.1.2	Main Location for distances between test objects	18

5.1.1.3	Main Locations for Vulnerable Road Users	18

5.1.1.4	Naming of Bus Signals	19

5.1.2	Additional Fine Locations	19

5.1.2.1	Fine Location 1	19

5.1.2.2	Fine Location 2	20

5.1.2.3	Fine Location 3	20

5.2	Channel codes for function based channels	20

5.2.1	TTC	20

5.2.2	Calculated vs. vehicle communication bus channels	21

5.3	Channel codes for distances	21

5.3.1	C2C distance channels	23

5.3.1.1	CCRs, CCRm, CCRb	23

5.3.1.2	CCFtap	23

5.3.2	VRU distance channels	24

5.3.2.1	Nearside, Farside, Longitudinal	24

5.3.2.2	Turning, Reverse	24

5.3.3	LSS distance channels	24

5.3.4	BSD distance channels	25

5.3.4.1	Overtaking	25

5.3.4.2	Merging	25

5.3.5	Path Error distance channels	26

5.3.5.1	VUT longitudinal path error	26

5.3.5.2	VUT lateral path error	27

5.3.6	Examples	27

5.4	Codes for Test Type C2C	28

5.5	Codes for Test Type VRU	30

5.6	Codes for Test Type LSS	32

5.7	Codes for Test Type DOW	32

6	Examples	34

6.1	Examples for MME 1.6	34

6.1.1	Example of a test information file MME 1.6	34

6.1.2	Example of a channel information file MME 1.6	36

6.1.3	Example of a channel data file MME 1.6	36

6.2	Examples for MME 2.1	38

6.2.1	Example of a test information file MME 2.1	38

6.2.2	Examples of object information files MME 2.1	38

6.2.3	Example of a channel information file MME 2.1	39

6.2.4	Example of a channel data file MME 2.1	40

# Normative references

The following referenced documents are necessary for the application of this document.

ISO 8855, Road vehicles — Vehicle dynamics and road-holding ability — Vocabulary

ISO 19206, Road vehicles — Test devices for target vehicles, vulnerable road users and other objects, for assessment of active safety functions —Part 1:Requirements for passenger vehicle rear-end targets

ISO 19206, Road vehicles — Test devices for target vehicles, vulnerable road users and other objects, for assessment of active safety functions —Part 2:Requirements for pedestrian targets

ISO 19206, Road vehicles — Test devices for target vehicles, vulnerable road users and other objects, for assessment of active safety functions —Part 4:Requirements for bicyclist targets

# Hints

For active safety tests a set of specific environments can be described in a general way. Recommendation for the usage of the MME format in the exchange of data between the test laboratories and the customers are summarized in the following pages.

The RED F is to be seen as enhancement to ISO 13499 in special requirement to active safety tests.

# MME Descriptors

## Remarks to MME descriptors for active safety

The following remarks are added to the existing MME descriptors which are defined as “Alphanumeric” without a remark in the ISO 13499 document:

| Field descriptor | Data format | Remark and values (List with semicolon or example) |
| --- | --- | --- |
| Class of test object X | Alpha-numeric | Type/class of the test object |
|  |  | “VUT", "GVT", "SOV" and additional Main Locations for VRU from RED F (e.g.: PDAD, PDAF, TWCA, TWSC,…) |
| Velocity int. TOB X | Float | Nominal longitudinal velocity of test object to determine validities |
|  |  | Used unit: m/s |
| Velocity intended lat. TOB x | Float | Nominal lateral velocity of test object to determine validities |
|  |  | Used unit: m/s |

## Additional MME descriptors for active safety

The following descriptors are an enhancement to the definition in ISO MME and cover the special requirements of active safety. If not specified in another way, all units are in SI units: m, m/s, m/s2

| Field descriptor | Data format | Remark and values (List with semicolon or example) |
| --- | --- | --- |
| Scenario | Alpha-numeric | Test scenario name |
|  |  | Values: see chapter “MME Attribute Values” |
| System Function | Alpha-numeric | Vehicle function under test |
|  |  | Values: see chapter “MME Attribute Values” |
| Test Campaign Type | Alpha-numeric | Campaign type to group tests |
|  |  | Values: see chapter “MME Attribute Values” |
| Test Campaign Name | Alpha-numeric | Campaign name to group tests |
| Type of data source | Alpha-numeric | Type of the test data source |
|  |  | Virtual Test; Physical Test |
| Region | Alpha-numeric | Region for the use of the vehicle under test |
|  |  | EU; UK |
| Robustness Layer | Alpha-numeric | See chapter “Robustness Layer” |
| Run repetition | Integer | Count of the run repetition; starting with 1 |
| Impact Location TOB X | Alpha-numeric | Location of the desired impact point in percentage or foot of the test object (striking point) [in % or ft ]; see for details chapter “Impact locations, heading and side definitions“ |
|  |  | Examples: -25; 0; 25; 50; 75; 100, 125 -2ft; 0ft; +2ft see also chapter “MME Attribute Values” |
| Test function side X | Alpha-numeric | test function side gives an information of the location of the target (VUT, main line, blind spot, …) according to values in fine location 1 |
|  |  | RI (right); LE (left); FR (front); RE (rear) |
| Velocity intended TOB x | Float | desired velocity for test object “x” (according to RED A v1.6.3) |
|  |  | Used unit: m/s |
| Velocity intended lat. TOB x | Float | desired lateral velocity for test object “x” (according to RED A v1.6.3) |
|  |  | Used unit: m/s |
| Velocity lateral TOB X | Float | lateral velocity for test object “x” (according to RED A v1.6.3) |
|  |  | Used unit: m/s |
| Acceleration int. TOB X | Float | Desired/specified acceleration or deceleration for test setup |
|  |  | Used unit: m/s |
| Acceleration int. lat. TOB X | Float | Desired/specified lateral acceleration or deceleration for test setup |
|  |  | Used unit: m/s |
| Lane Departure Side TOB X | Alpha-numeric | Lane Departure Side of TOB X during test |
|  |  | Values: Driver¦Passanger¦NOVALUE |
| Headway distance int. TOB X | Float | Desired/specified headway distance for test start |
| Dist. Offset ini. Pos. TOB X | Float | Desired/specified acceleration or deceleration for test setup |
| Time Offset ini. Pos. TOB X | Float |  |
| Intended Heading TOB X | Integer | Desired/nominal/specified heading of the test object at test start ; see for details chapter “Impact locations, heading and side definitions“ |
|  |  | Used unit: degree |
| Lane marking function line | Alpha-numeric | Documentation of lane marking on function side |
|  |  | Any (comment: continuous line marking of not specified lane marking or several lane markings); Bot Dots; Dashed; Solid; none (comment: no line marking); Undefined (comment: line marking available ins sections: interrupted marking or different marking in sections) |
| Lane marking secondary line | Alpha-numeric | Documentation of lane marking on secondary side |
|  |  | Any (comment: continuous line marking of not specified lane marking or several lane markings); Bot Dots; Dashed; Solid; none (comment: no line marking); Undefined (comment: line marking available ins sections: interrupted marking or different marking in sections) |
| Vel. int. lane dep. TOB X | Float | Desired/specified lane departure velocity for the test setup |
|  |  | Used unit: m/s |
| Distance eye to end TOB X | Float | Distance from the end of the vehicle to the center of the driver's eye ellipse (e.g. 95% Percentile for CNCAP 2021) |
|  |  | Used unit: m |
| Environment day light | Alpha-numeric | day light |
|  |  | day; night; street lights; headlight glare |
| Road surface temperature | Float | road surface temperature |
|  |  | Used unit: K |
| Sky conditions | Alpha-numeric | Sky / Cloudiness |
|  |  | cloudless; slightly cloudy; cloudy; not visible (in German: wolkenlos; leicht bewoelkt; bewoelkt; nicht erkennbar) |
| Precipitation type | Alpha-numeric | description of precipitation |
|  |  | none; rain; snow; hail (in German: kein; Regen; Schnee; Hagel) |
| Precipitation intensity | Alpha-numeric | intensity of precipitation type (in German: Niederschalgsintenität) |
|  |  | not relevant; low; medium; strong (in German: nicht relevant; niedrig; mittel; stark) |
| Weather obstruction | Alpha-numeric | description of obstruction by weather |
|  |  | fog; smog; spray; none (in German: Nebel; Smog; Gischt; keine) |
| Weight of obstruction | Alpha-numeric | intensity of weather obstruction |
|  |  | not relevant; low; medium; strong (in German: nicht relevant; niedrig; mittel; stark) |
| Wind speed | Float | wind speed |
|  |  | Used unit: m/s |
| Road conditions | Alpha-numeric | properties of road conditions od street or proving ground |
|  |  | dry; wet; wet; snowy; icy; NOVALUE (in German: trocken; nass; feucht; schneebedeckt; vereist; NOVALUE) |
| Shape Front TOB X | Tuples | the count starts on front right side; If the number of tuples is odd, the middle one must lie on the centerline of the vehicle; This center point is (0;0) |
|  |  | Used unit: m |
|  |  | Format: (x1;y1) (x2;y2) (x3;y3) (x4;y4) (x5;y5) (x6;y6) (x7;y7) |
| Shape Rear TOB X | Tuples | the count starts on rear right side; If the number of tuples is odd, the middle one must lie on the centerline of the vehicle |
|  |  | Used unit: m |
|  |  | Format: (x1;y1) (x2;y2) (x3;y3) (x4;y4) (x5;y5) (x6;y6) (x7;y7) |
| Shape Right Side TOB X | Tuples | the count starts on the front; |
|  |  | Used unit: m |
|  |  | Format: (x1;y1) (x2;y2) (x3;y3) (x4;y4) (x5;y5) (x6;y6) (x7;y7) |
| Shape Left Side TOB X | Tuples | the count starts on the front; |
|  |  | Used unit: m |
|  |  | Format: (x1;y1) (x2;y2) (x3;y3) (x4;y4) (x5;y5) (x6;y6) (x7;y7) |
| Dimensions TOB X | Tuples | length and width as tuple |
|  |  | Used unit: m |
|  |  | (length; width) |
| Front overhang TOB X | Float | Length of the front overhang of test object in m |
| Driver State TOB X | Alpha-numeric | Desired state of the driver for the scenario |
|  |  | NOVALUE; Attentive; Inattentive |
| S/W version TOB X | Alpha-numeric | Description of used S/W in TOB X |
| Test completion | Alpha-numeric | Status of test completion |
|  |  | NOVALUE; Completed; Aborted; Interaction |
| Position of the sun | Tuples | Proposal for data type: string Data format: (α; β) with α° as angle to VUT main direction β° as angle to proving ground |
|  |  | (α; β) |
| Glare angle | Float | Glare angle in degrees |

## MME Attribute Values

### Test object

In active safety tests the interaction between 2 or more test objects are described. In all scenarios the subject vehicle is engaged, which test object code is 1.

The other objects could be targets like vehicles, vulnerable road users and/or parts of the test rig like lines. The test object code for all targets is 2.

If additional targets, obstruction vehicles or other vehicles are used in the test, the test object codes will be counted continuously. In the count, all targets come before the concealment vehicles.

In most cases the test object T for the test rig resp. testing ground is not explicitly named or listed.

### System Function

Description of the tested system function of the vehicle.

| Function | Comment |
| --- | --- |
| ACC | Adaptive Cruise Control |
| AD | Assisted Driving |
| ADDW | Advanced Driver Distraction Warning |
| AEB | Autonomous Emergency Braking |
| AES | Autonomous Emergency Steering |
| AIFM | Advanced Impaired-driving Monitoring |
| BSD | Blind Spot Detection |
| CIB | Crash Imminent Braking |
| DBS | Dynamic Brake Support |
| DMS | Driver Monitoring System |
| DOW | Door Opening Warning |
| ELK | Emergency Lane Keeping |
| ESA | Emergency Steering Assist |
| ESS | Emergency Steering Support |
| FCW | Forward Collision Warning |
| FP | False Positive |
| HBA | High Beam Assist |
| ISA | Intelligent Speed Assistance |
| LCA | Lane Change Assist |
| LCC | Lane Centering Control |
| LDW | Lane Departure Warning |
| LKA | Lane Keeping Assist |
| RCTA | Rear Cross Traffic Alert |
| RCW | Rear Collision Warning |
| SAS | Speed Assistance System |
| SLIF | Speed Limit Information Function |
| TSR | Traffic Sign Recognition |

### Scenario

Description of the scenario setup in testetin

| Scenario | Comment |
| --- | --- |
| ACC | Adaptive Cruise Control |
| ACPE | Accelerator Control for Pedal Error |
| BSM | Blind Spot Monitoring |
| C2C SCP | Car-to-Car Straight Crossing Path |
| C2C SCPO | Car-to-Car Straight Crossing Path with Obstruction |
| CBDA | Car-to-Bicyclist Dooring Adult |
| CBFA | Car-to-Bicyclist Farside Adult |
| CBLA | Car-to-Bicyclist Longitudinal Adult |
| CBNA | Car-to-Bicyclist Nearside Adult |
| CBNAO | Car-to-Bicyclist Nearside Adult Obstructed |
| CBTA | Car-to-Bicyclist Turning Adult |
| CCCscp | Car-to-Car Crossing Straight Crossing Path |
| CCFhol | Car-to-Car Front Head-On Lane Change |
| CCFhos | Car-to-Car Front Head-On Straight |
| CCFT | Car-to-Car Front Turn-Across-Path |
| CCFtap | Car-to-Car Front Turn Across Path |
| CCRb | Car-to-Car Rear Braking |
| CCRci | Car-to-Car Rear Cut-in |
| CCRco | Car-to-Car Rear Cut-out |
| CCRH | Car-to-Car Rear Stationary |
| CCRm | Car-to-Car Rear Moving |
| CCRs | Car-to-Car Rear Stationary |
| CMCscp | Car-to-Motorcyclist Crossing Straight Crossing Path |
| CMFtap | Car-to-Motorcyclist Front Turn Across Path |
| CMRb | Car-to-Motorcyclist Rear Braking |
| CMRci | Car-to-Motorcyclist Rear Cut-in |
| CMRco | Car-to-Motorcyclist Rear Cut-out |
| CMRs | Car-to-Motorcyclist Rear Stationary |
| CPFA | Car-to-Pedestrian Farside Adult |
| CPLA | Car-to-Pedestrian Longitudinal Adult |
| CPMFC | Car-to-Pedestrian Manoeuvring Front Child |
| CPMRCm | Car-to-Pedestrian Manoeuvring Reverse Child Moving |
| CPMRCs | Car-to-Pedestrian Manoeuvring Reverse Child Stationary |
| CPN | Car-to-Pedestrian Nearside Child Obstructed |
| CPN | Car-to-Pedestrian Nearside Child Obstructed |
| CPNA | Car-to-Pedestrian Nearside Adult |
| CPTA | Car-to-Pedestrian Turning Adult |
| DA-CI | Driving Assistance – Cut-In |
| DA-OT | Driving Assistance – Overtaking |
| DOW | Door Opening Warning |
| DSM | Driver State Monitoring |
| ELK-ON | Emergency Lane Keeping - Oncoming |
| ELK-OT | Emergency Lane Keeping –Overtaking |
| ELK-RE | Emergency Lane Keeping – Road Egde |
| iACC | Intelligent Adaptive Cruise Control |
| ISL | Intelligent Speed Limiter |
| LC | Lane Centering |
| LCA | Lane Change Assist |
| LDW | Lane Departure Warning |
| LKA | Lane Keeping Assist |
| RCTAp | Rear Cross Traffic Alert for perpendicular parking |
| RCTAd | Rear Cross Traffic Alert for diagonal parking |
| RCTBp | Rear Cross Traffic Braking/Prevention for perpendicular parking |
| RCTBd | Rear Cross Traffic Braking/Prevention for diagonal parking |
| M | Merging |
| On | Oncoming |
| Ov | Overtaking |
| OvF | Overtaking farlane |
| OI | Overtaking Intentional |
| OU | Overtaking Unintentional |
| OIF | Overtaking Intentional Farlane |
| OUF | Overtaking Unintentional Farlane |
| SCF | Speed Control Function |
| SLF | Speed Limitation Function |
| SLIF | Speed Limit Information Function |
| SLWF | Speed Limit Warning Function |
| VATS | Visual Attention Time Sharing |

### Test Campaign Type

The given list of types of tests focuses on the description of the test set-up in terms of the test objects, the procedure and the test track. The necessary measuring equipment also plays a role in the grouping in order to cluster similar tests.

| Value | Description | Test objects |  | Remark |
| --- | --- | --- | --- | --- |
|  |  | Code | Class |  |
| BSD | Blind Spot Detection | 1 2 | Subject Vehicle Vehicle Target or other target | Several lanes and road markings required |
| C2C | Car to Car | 1 2 | Subject Vehicle Vehicle Target |  |
| DHL | Dynamic Head Light | 1 | Subject Vehicle |  |
| LSS | Lane Support Systems | 1 | Subject Vehicle | Various types of road markings required |
| OSM | Occupant Status Monitor | 1 | Subject Vehicle | Special remark to occupants |
| SAS | Speed Assist Systems | 1 | Subject Vehicle |  |
| VRU | Vulnerable Road User | 1 2 | Subject Vehicle Target: Pedestrian, Cyclist Target, etc. |  |
| DIV | Diverse test scenarios | Undef. | Undef. |  |

### Type of the test

The type of test can be uses to describe the test setup.

It can be a combination of system function and scenario in addition with further details.

### Subtype of the test

The subtype of the test can be uses to describe the test setup in more detail.

In many cases, it is helpful to specify the speed in km/h varied in the scenario as the first/only value in the "subtype of the test":

| subtype of the test | Comment |
| --- | --- |
| st | Straight |
| cu | Curve |
| D | Day |
| N | Night |
| fs | Far Side |
| ns | Near Side |
| fo | Far Side Obstructed |
| no | Near Side Obstructed |
| FP | FalsePositive |
| diag | diagonal parking |
| RG0.5 | Corresponding rear gap 0.5m / L1 |
| RG1 | Corresponding rear gap 0.1m / L2 |
| RG1.5 | Corresponding rear gap 1.5m / L3 |
| RG2 | Corresponding rear gap 2.0m / L4 |
| RG5 | Corresponding rear gap 2.5m / L5 |
| U | Unintentional |
| I | intentional |
| Max-Creep | Creeping to Maximum Speed Before Acceleration /D1 |
| 1s-Crep | One-Second Creeping Before Acceleration /2 |
| perpend | perpendicular parking |

The values for ‘Subtype of the test’ can be expanded by the exchanging partners. The table contains recommendation for a subset of the types of test listed in 3.2.

The Subtype of the test is a composition of scenario and function.

### Regulation

The value of the descriptor regulation in the MME file is a comma separated list of keys each composed by 3 parts separated by ‘_’ (ASCII 95).

The first part describes the Family which represents a group of regulations or procedures like UN-R, GTR, EuroNCAP.

The second is the subpart which could be the Number of the regulation or an abbreviation of the procedure like 127, VRU, LSS.

The third part gives a specification of the Version number or year of publication/entry into force like 8.7, IV, 2022. If the year format was selected and this is no longer unambiguous due to an update or requires a distinction, the month can be added using the syntax "-MM".

A list of examples gives the following table.

| Value | Remark |
| --- | --- |
| EuroNCAP_CAFC_2026 | Euro NCAP 2026 - Crash Avoidance - Frontal Collisions 1.1 |
| EuroNCAP_CALDC_2026 | Euro NCAP 2026 - Crash Avoidance - Lane Departure Collisions 1.1 |
| EuroNCAP_CALSP_2026 | Euro NCAP 2026 - Crash Avoidance - Low Speed Collisions v1.1 |
| EuroNCAP_SDON_2026 | Euro NCAP 2026 - Safe Driving - Occupant Monitoring v1.1 |
| EuroNCAP_SADE_2026 | Euro NCAP 2026 - Safe Driving - Driver Engagement v1.1 |
| EuroNCAP_SDVA_2026 | Euro NCAP 2026 - Safe Driving - Vehicle Assistance v1.1 |
| USNCAP_DBS_2015 | Version from 2015 of the NHTSA test protocol for dynamic brake support |
| CIASI_C2C_2017 | Version from 2017 of the CIASI test protocol for car to car scenarios |
| ANCAP_LSS_3.0.2 | Version 3.0.2 of the ANCAP test protocol for lane support systems |
| IIHS_VRU_II | Version II of the pedestrian AEB test protocol of the IIHS - or alternatively: |
| IIHS_VRU_2019 | Version II from 2019 of the pedestrian AEB test protocol of the IIHS |
| … | … |

### Name of test object X

| Name TOB | Description |
| --- | --- |
| VUT | Vehicle Under Test |
| GVT | Global Vehicle Target |
| RVT | Real Vehicle Target |
| EPTa | Euro NCAP Pedestrian Target adult |
| EPTc | Euro NCAP Pedestrian Target child |
| EBTa | Euro NCAP Bicyclist Target adult |
| EMT | Euro NCAP Motorcyclist Target |
| RMT | Real Motorcyclist Target |
| SOV | Suspect Other Vehicle |
| OV | Other Vehicle |
| POV | Principal Other Vehicle |
| ATV | Approaching Target Vehicle |
| LTV | Lead Target Vehicle |
| FTV | Following Target Vehicle |
| OLV | Oncoming Lane Vehicle |
| SV | Stationary Vehicle |
| MV | Moving Vehicle |
| TV | Target Vehicle |
| PTAx | Pedestrian Target Adult x% |
| PTCx | Pedestrian Target Child x years old |
| CTx | Cyclist Target x% |
| MT | Motorcycle Target |
| E-Scooter | E-Scooter Target |
| Stroller | Pedestrian with Stroller Target |
| Wheelchair | Wheelchair User Target |
| Toy | Toy (e.g.: Ride-On Car, Running Bike, Tricycle, Self-balancing Scooter) |
| Animal | Animal Target |
| ObstructionSmall | Small Obstruction Vehicle |
| ObstructionLarge | Large Obstruction Vehicle |

### Vehicle Shape and Vehicle Dimension

Vehicle width – Maximum vehicle including the door handles. The rear-view mirrors, side marker lamps, tyre pressure indicators, direction indicator lamps, position lamps, flexible mud-guards and the deflected part of the tyre side-walls immediately above the point of contact with the ground are neglected for the width.

Vehicle length – Distance from the foremost point to the rearmost point on the vehicle centreline.

### Impact locations, heading and side definitions

The impact locations for vehicles and targets are measured in % of the dimension. The 0% value is always determined based on the natural direction of movement of the test object. The percentage increases with the direction of movement and from left to right, based on the natural direction of movement.

For further details and possible values please see table 1 and figure 1 below.

| Attribute Name | Comment​ |
| --- | --- |
| Impact location TOB X​ | [%] from right to left / from rear to front (value in % without unit “%”)​ |
| Impact side test object X​ | FR, RE, NS, FS, CG, RI, LE​ |
| Intended Heading TOB X​ | Main TOB (VUT) starts with 0° Heading over the proving ground. All other TOBs have an intended heading direction derived from this direction [°] |
| Test function side TOB X​ | FR, RE, NS, FS, LE, RI​ |

For non-vehicle targets with variable contour during test, the possible impact point is described as “GC” I addition with the impact side.

### Robustness Layer

The robustness layer is to verify ADAS system performance on non ideal test conditions, but also under realistic and reproduceable variations.

| Type | Robustness Layer Name | Code | Parameter | Example |
| --- | --- | --- | --- | --- |
| None | Not applicable | NOVALUE | No robustness layer applied |  |
| VUT | Driver input pre-crash | DI |  |  |
|  | Impact location | IL | adjusted* impact location in % |  |
| Target | Speed | S | adjusted* target velocity in m/s | Variations in target speed |
|  | Acceleration | A | adjusted* target acceleration in m/s2 | Variations in target acceleration |
|  | Initial position offset | IP | adjusted* target position offset in m | Variations in starting point of test objects |
|  | Trajectory/Heading | H | adjusted* target heading in ° | Variations in heading of test objects |
| Environment | Illumination (Nighttime) | N |  | night-time pedestrian detection |
|  | Illumination (Headlamp Glare) | HG |  | Oncoming headlights |
|  | Infrastructure/Clutter | I |  | Parked vehicle is in front of pedestrian path |
|  | Obscuration/Obstruction | O |  | Stationary pedestrian blocking view to walking pedestrian path |

* Relative to the intended scenario parameter without the robustness layer applied: A robustness of 5km/h (= 1,39m/s) for target speed is written as “Robustness Layer: Target, S, -1.39”

Variation for values for robustness is according to following rules:

- Speed and acceleration reductions relative to the nominal scenario value are expressed as negative values.

- Smaller impact location values correspond to a shift of the impact location towards the beginning of the scale (0%).

- Heading variations follow the right-hand rule around the rotation axis. Positive values represent anti-clockwise rotations, while negative values represent clockwise rotations.

# Reference Coordinate Systems

All channels are measured or calculated in specific reference coordinate systems. For MME 2.x the coordinate system Id is part of the extended channel code. The related reference system has to be agreed between the exchanging partners. The reference systems used in active safety tests can be reduced to the following table.

| Id | Characteristic | Directions | Description | Remark |
| --- | --- | --- | --- | --- |
| 1DY | SV dynamic | X Y Z | Dynamic coordinate system fixed to the SV according to ISO 8855 | Vehicle coordinate system according ISO 8855. Moving direction is X |
| 2DY | Target dynamic | X Y Z | Dynamic coordinate system fixed to the target according to ISO 8855 | Analog to a vehicle coordinate system according ISO 8855. Moving direction is X |
| 1DF | SV dynamic with orientation fixed over TST | 1 2 3 | Dynamic coordinate system with orientation fixed to TST fixed to the moving SV | Vehicle coordinate system based on 1DY on test start. Moving direction is 1 |
| 2DF | Target dynamic with orientation fixed over TST | 1 2 3 | Dynamic coordinate system with orientation fixed to TST fixed to the moving target | Vehicle coordinate system based on 2DY on test start. Moving direction is 1 |
| LOC | Steering Wheel | 1 2 3 | Local Coordinate system of the steering wheel (1 = Longitudinal) | Only the rotation around the longitudinal axis of the steering system is used. |
| LOC | Path System | X Y Z | Local coordinate system of the path | Only the lateral deviation from the path is used. |
| NED | NorthEastDown | 1 2 3 | Stationary earth-fixed axis system (1 = North, 2 = East, 3 = Down) | Typically from GPS based systems with units [m]. |
| TST | Testground Static | X Y Z | Stationary earth-fixed axis system with an origin that is fixed in the ground plane | Derived from NED system by moving the origin to a point at the test ground and rotating the X axis to the main driving direction. |

1DY and 2DY are right-hand coordinate systems according to ISO 8855, where: .

The X and Y axes of the coordinate system are parallel to the ground plane, with the X and Y axis aligned with the vertical projection of the XV and YV axis (vehicle axis) on to the ground plane.

The vehicle reference point is the point fixed in the vehicle sprung mass if no other point is explicit named. But this vehicle reference point may be defined in a variety of locations, based on the needs of the analysis or test: vehicle center of gravity, the sprung mass center of gravity, the mid-wheelbase point at the height of the center of gravity, and the center of the front axle.

1DF and 2DF are right-hand coordinate systems according to ISO 8855, where: .  1DF and 2DF are based on 1DY/2DY with fixed orientations of the axis to the initial direction over test ground. Despite all rotations of the test object over the test ground, the coordinate system maintains its orientation in X and Y.

In order to show that only longitudinal and lateral distances are determined, the directions 1, 2 and 3 are used.

## Vehicle Coordinate System

The vehicle coordinate system is specified in ISO 855:2011. The origin of the coordinate system is located in the middle of the vehicle front. So the middle of the centerline is (0; 0; z), where z is specific to the vehicle height.

Z

    Yaw ψ

X
      Roll φ

Z

Pitch θ

## Test Ground Coordinate System

The coordinate system of the test ground is a right-hand coordinate system where the x-axis is aligned with the initial direction of travel of the VUT. The y-axis is perpendicular to the x-axis.

The origin of this coordinate system is defined by scenario / type of test. This may be the expected collision point or the intersection between the projected paths of test objects.

# Channel Codes

The channel code consists of 16 characters, composed of a sequence of codes with a fixed length and specific order, defining test object, position, main location, fine locations, physical dimension, direction and filter class

| EXAMPLE | 1 0 VEHC 00 DI 00 VE X P |  |  |  |
| --- | --- | --- | --- | --- |
| Meaning: | Test object | = 1 | Subject Vehicle |  |  |
|  | Position | = 0 | Undefined |  |  |
|  | Main location | = VEHC | The whole vehicle |  |  |
|  | Fine location 1 | = 00 | Undefined |  |  |
|  | Fine location 2 | = DI | Difference (relative) |  |  |
|  | Fine location 3 | = 00 | Undefined |  |  |
|  | Dimension | = VE | Velocity |  |  |
|  | Direction | = X | X-direction |  |  |
|  | Filter class | = P | Prefiltered |  |  |

No filter classes are defined for active safety signals. Therefore all channels which could be filtered should have the filter class P (Prefiltered). Data sets which not could be filtered like Event or Time Channels should have the filter class 0. The filter class has to be S (Special) when Acceleration, Force or Angular Velocity Channels are filtered according to the specification given in the regulation.

## Enhancement for specific Main and Fine Locations for active safety

### Additional Main Locations

As ISO MME was focus on passive safety there were main location defined to describe parts of a vehicle. Active safety requires a more function based approach. For this purpose are the following main locations as specific extension defined.

#### Function based Main Locations

| Main Location | Description |
| --- | --- |
| TTTC | AEB Time-to-Collision |
| TAEB | AEB activation time |
| TFCW | FCW activation time |
| TIMP | Impact time |
| TECS | Time where VUT enters in curve segment |
| TLKA | LKA activation time |
| TLDW | LDW activation time |
| TLCR | Line crossing time |
| TDOP | Time door opening |
| TRCT | Time Rear Crossing Traffic |
| THMS | Time Hold Management System |
| TABS | Time ABS status |
| TSAS | Time of Speed Limiter Action |

#### Main Location for distances between test objects

| Main Location | Description |
| --- | --- |
| DSOV | Distance SOV |
| DGVT | Distance GVT |
| DCOL | Collision Point distance |
| DLIN | Distance to Line |
| DINT | Distance to Intersection |
| DVUT | Distance to VUT |
| DIST | other Distance |

#### Main Locations for Vulnerable Road Users

Test devices for targets, vulnerable road users and other objects are described in ISO 19206 Part 1, 2 and 4. Especially for vulnerable road users the number of different targets will grow in the future.

Here is a list of ISO Main Locations for these targets:

|  | Group | Type | ISO ML code | Description | Comment |
| --- | --- | --- | --- | --- | --- |
| Animals | AN | MS | ANMS | Moose |  |
|  | AN | RO | ANRO | Roe |  |
|  | AN | WB | ANWB | Wild boar |  |
|  | AN | DR | ANDR | Deer |  |
|  | AN | HS | ANHS | Horse |  |
|  | AN | CW | ANCW | Cow |  |
|  | AN | DG | ANDG | Dog |  |
|  | AN | CT | ANCT | Cat |  |
|  | AN | BB | ANBB | Big Bird | walking Bird (e.g. Duck, Chicken, Goose,…) |
| Toys | TY | RC | TYRC | Ride-On Car | = e.g. Bobby Car |
|  | TY | RB | TYRB | Running Bike |  |
|  | TY | TC | TYTC | Tricycle |  |
|  | TY | SS | TYSS | Self-balancing Scooter | = e.g. Hoverboard or Hover Scooter |
| Two Wheeler | TW | CA | TWCA | Cyclist Adult |  |
|  | TW | CC | TWCx | Cyclist Child x years old | tbd: replace x by age when target is available |
|  | TW | EB | TWEB | Electric Cyclist / Pedelec | EuroNCAP target PTW |
|  | TW | MB | TWMB | Motorbike / PTW | EuroNCAP target PTW |
|  | TW | SC | TWSC | Scooter Adult | CNCAP e-Scooter or Motor scooter |
|  | TW | KS | TWKS | Kick Scooter | adult rider on (electric) kick scooter |
|  | TW | Kx | TWKx | Kick Scooter Child x years old | child rider on (electric) kick scooter tbd: replace x by age when target is available |
|  | TW | SW | TWSW | Segway or similar |  |
|  | TW | WC | TWWC | Wheelchair |  |
| Pedestrian | PD | AD | PDAD | Adult / Adult Midsize 50% |  |
|  | PD | AF | PDAF | Adult Female 5% |  |
|  | PD | AL | PDAL | Adult Large 95% |  |
|  | PD | Cx | PDCx | Child x years old | tbd: replace x by age when target is available |
|  | PD | C1 | PDC1 | Child 1 year old | existing target PDCx |
|  | PD | C2 | PDC2 | Child 2 years old | existing target PDCx |
|  | PD | C7 | PDC7 | Child 7 years old | existing target PDCx |
|  | PD | AW | PDAW | Adult Senior with 4-wheeled walker |  |
|  | PD | BS | PDBS | Adult with baby stroller |  |
|  | PD | SB | PDSB | Skateboarder |  |
|  | PD | IS | PDIS | Inline Skater |  |

#### Naming of Bus Signals

Bus signals can be named with the main location 2BUS.

Since many bus channels measure quantities that are also recorded by reference measurement technology, the main location of the measured quantity can be used with a specific value in the fine location 3. When bus signals are equivalent to named channels the code of the named channel with the Fine location 3 CB (car communication bus).

### Additional Fine Locations

#### Fine Location 1

| FL | long description | comment |
| --- | --- | --- |
| WA | main warning signal | main warning signal as reaction for a system/function given by main location |
| PW | pre-warning signal | prewarning signal as reaction for a system/function given by main location |
| BE | braking event | breaking event as reaction for a system/function given by main location |
| PB | pre-braking event | prebreaking event as reaction for a system/function given by main location |
| ST | Status | current system status (e.g. on/off) |
| DC | dynamic calculation | Addition to TTTC for dynamic calculation of current speed/acceleration (für US-NCAP - Euro-NCAP assumes continuous travel) to determine a difference between EuroNCAP ("...would continue to travel with the speed it is travelling.") and US-NCAP (with acceleration) |
| PE | Path Error |  |
| SH | Shape |  |
| BX | Boundary Box |  |
| RC | Rotation Center |  |
| TY | Tyre |  |
| AX | Axle |  |
| EY | Eye-Position |  |
| DR | Door |  |
| BP | Bumper |  |

#### Fine Location 2

| FL | long description | comment |
| --- | --- | --- |
| RQ | Requested | Indicator for Request on CAN bus or action request like "door opening interface" |
| SR | Switching Release | Indicator for Release of an switching request on CAN bus or action request |
| AC | acoustical | detection of accoustical signal |
| OP | optical | detection of optical signal |

#### Fine Location 3

| FL | long description | comment |
| --- | --- | --- |
| CB | communication bus (e.g. CAN) | Analysis of channel via CAN bus signal and electric/optical measurement |
| SH | Shape |  |
| BX | Boundary Box |  |
| RC | Rotation Center |  |
| TY | Tyre |  |
| AX | Axle |  |
| EY | Eye-Position |  |
| DR | Door |  |
| BP | Bumper |  |

## Channel codes for function based channels

### TTC

For calculation and evaluation the following channels are used for TTC:

- 10TTTC000000TI00   = TTC

- 10TTTCDC0000TI00  = TTC_with_Acc

As TTC is a calculated channel, the data is

If the laboratory decides to calculate TTC an add it to the data, the following channel codes are reserved:

- 101TTC000000TI00   = TTC

- 101TTCDC0000TI00  = TTC_with_Acc

The customer TTC channels are stored with main location 2TTC:

- 102TTC000000TI00   = TTC

- 102TTCDC0000TI00  =  TTC_with_Acc

### Calculated vs. vehicle communication bus channels

When measurement data is recorded on the the vehicle bus system, the channel is named according to the recorded size according to the naming scheme, with the letters CB for "communication bus" inserted at Fine Location 3.

## Channel codes for distances

The most distances which must be known in active safety tests are between moving test objects and not located on one of these. There is a kneed to describe the involved test objects and also the used location on the test objects for each distance. So an enhancement for ISO codes is necessary.

The channel codes for distances between test object use TO, SP and FL1 to describe the first test object and FL2 and FL3 the second test object. The first object in this case is the test object, which is more important in this range: VUT, VRU, GVT, SOV.

| TO | SP | ML | FL1 | FL2 | FL3 | Dimens. | Direct. | Filter | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | DGVT | MI | 22 | MI | DC | 1 | P | longitudinal distance of front middle VUT to front middle GVT |
| Fist TO |  |  | Fist TO | Second TO |  |  |  |  |  |

FL2 is used for the second test object like the values TO+SP for the first test object.

| TO | First/more important test Object (definition of VUT, GVT,… is in MME file) |
| --- | --- |
| SP | Location on first/more important test object (VUT, Target, GVT,…) |
| ML | Description of distance location |
| FL1 | location on first object (TO + SP) ( MI = middle; LE = left side without mirror; BX = Boundary Box; SH = Shape) |
| FL2 | TO + SP for second test object (e.g. 38 = TO3 (SOV) rear middle) |
| FL3 | location on second test object ( MI = middle; LE = left side without mirror; BX = Boundary Box; SH = Shape) |
| Dimens. | Dimension of the channel (for distances this will be DC) |
| Direct. | Direction of distance |
| Filter | Filter class |

The following list shows all possible values for the positions in channel codes for distances:

| TO | Versuchsobjekt / test object |  |  | SP |  |  |  | ML |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | VUT |  |  | 0 | undefiniert / overall |  |  | DSOV | Distance SOV |
| 2 | GVT / Target |  |  |  |  |  |  | DGVT | Distance GVT |
| 3 | ggf. SOV |  |  |  |  |  |  | DCOL | Collision Point distance |
| X | running no. of Dummy |  |  |  |  |  |  | DLIN | Distance to Line |
|  |  |  |  |  |  |  |  | DINT | Distance to Intersection |
|  |  |  |  |  |  |  |  | DVUT | Distance to VUT |
|  |  |  |  |  |  |  |  | DIMP | Distance to impact point |
|  |  |  |  |  |  |  |  | DIST | other Distance |
|  |  |  |  |  |  |  |  | VEHC | vehicle |

| FL1 |  |  | FL2 | TO + SP for GVT / SOV / Target | FL3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SH | Shape | VUT / GVT / SOV | 2x | GVT + descr. of SP of MME | SH | Shape | VUT / GVT / SOV |
| BX | Boundary Box |  | 3x | SOV + descr. of SP of MME | BX | Boundary Box |  |
| FR | Front |  | 21 | GVT front left | FR | Front |  |
| UP | Oben |  | 20 | VRU target | UP | Up |  |
| MI | Middle |  | … | … | MI | Middle |  |
| CG | Center of Gravity |  |  |  | CG | Center of Gravity |  |
| RC | Rotation Center |  | FL 2 | For Dummies: No. Of test object + impact side | RC | Rotation Center |  |
| RE | Rear |  |  |  | RE | Rear |  |
| LE | Left |  |  |  | LE | Left |  |
| RI | Right |  | 22 | front | RI | Right |  |
| TY | Tyre |  | 24 | left side | TY | Tyre |  |
| AX | Axle |  | 20 | undef. | AX | Axle |  |
| EY | Eye-Position |  | 25 | nearest side to VUT | EY | Eye-Position |  |
| DR | Door |  |  |  | DR | Door |  |
| BP | Bumper |  |  |  | BP | Bumper |  |
| 00 | undef. |  |  |  | 00 | undef. |  |
|  |  |  |  |  | FR | Front | Dummies |
|  |  |  |  |  | LE | Left |  |
|  |  |  |  |  | RI | Right |  |
|  |  |  |  |  | RE | Rear |  |
|  |  |  |  |  | CG | Center |  |
|  |  |  |  |  | IN | Inner | Line |
|  |  |  |  |  | OU | Outer |  |

| Direction |  | coordinate system |
| --- | --- | --- |
| 0 | Undefined/other |  |
| 1 | Longitudinal | 1DT |
| 2 | Lateral | 1DT |
| 3 | Vertical | 1DT |
| X | Longitudinal | 1DY |
| Y | Lateral | 1DY |
| Z | Vertical | 1DY |
| 0 | Resultant |  |
| The coordinate system refers to test object in TO |  |  |

### C2C distance channels

#### CCRs, CCRm, CCRb

12DGVTMI28MIDC1P – longitudinal Distance Front VUT- Back GVT

12DGVTMI28MIDC2P – lateral Distance Front VUT- Back GVT

12DGVTMI28REDCXP – Headway Distance Front VUT- Back GVT (in X)

#### CCFtap

11DGVTSH21BXDC1P – C2C longitudinal Distance Front Shape VUT – GVT Box

11DGVTSH21BXDC2P – C2C lateral Distance Front Shape VUT – GVT Box

12DGVTMI20BXDCXP – C2C Headway Distance  Front Shape VUT – GVT Box (in X)

### VRU distance channels

#### Nearside, Farside, Longitudinal

12DVUTSH20MIDC1P – VUT-VRU longitudinal Distance

(0 instead of 2/4/6/8  same code for nearside and farside)

12DVUTMI20MIDC2P – VUT-VRU lateral Distance

12DVUTMI20MIDC0P – VUT-VRU resultant distance

#### Turning, Reverse

12DVUTMI20MIDC1P – VRU longitudinal Distance

12DVUTMI20MIDC2P – VRU lateral Distance

12DVUTMI20MIDC0P – resultant distance

18DVUTMI20MIDC1P – VRU longitudinal Distance

18DVUTMI20MIDC2P – VRU lateral Distance

### LSS distance channels

13DLINTY00INDC2P - lateral distance Tyre to line crossing (inner edge)

13DLINTY00OUDC2P - lateral distance Tyre to line crossing (outer edge)

### BSD distance channels

#### Overtaking

#### Merging

### Path Error distance channels

#### VUT longitudinal path error

10VEHCPE0000DC1P = XVUT_desired - XVUT_actual

XVUT_actual: actual X-Coordinate of middle front of the VUT
                 in relation to GVT

XVUT_desired: desired X-Coordinate of middle front of the VUT
                 in relation to GVT

The longitudinal path error is the difference between actual distance VUT-GVT and desired distance VUT-GVT

*CCFtab+CMFtab: Instead of GVT a fixed point before the collision point is used:

#### VUT lateral path error

10VEHCPE0000DC2P = YVUT_error + YGVT_error

CMFtab: The path error is calculated by YVUT_error

10VEHCPE0000DC2P = YVUT_error

### Examples

| TO | SP | ML | FL1 | FL2 | FL3 | Dim. | Direct. | Filter | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | DGVT | MI | 22 | MI | DC | 1 | P | longitudinal distance of front middle VUT to front middle GVT |
| 1 | 2 | DGVT | MI | 22 | MI | DC | 2 | P | lateral distance of front middle VUT to front middle GVT |
| 1 | 2 | DGVT | MI | 22 | MI | DC | 0 | P | absolute distance of front middle VUT to front middle GVT |
| 1 | 2 | DSOV | LE | 32 | LE | DC | 1 | P | longitudinal distance of front middle VUT to front middle SOV |
| 1 | 4 | DSOV | LE | 34 | LE | DC | 2 | P | lateral distance of front middle VUT to front middle SOV |
| 2 | 2 | DSOV | MI | 38 | MI | DC | 1 | P |  |
| 1 | 3 | DLIN | TY | 00 | IN | DC | 2 | P | VUT tyre to inner side of line |
| 1 | 3 | DLIN | TY | 00 | OU | DC | 2 | P | VUT tyre to outer side of line |
| 1 | 2 | DGVT | MI | 28 | MI | DC | 1 | P | Headway distance (lateral) |
| 1 | 2 | DGVT | MI | 28 | MI | DC | 2 | P | lateral Distance (Middle to Middle) |
| 1 | 2 | DGVT | MI | 28 | RE | DC | X | P | Headwy distance in X |
| 1 | 1 | DGVT | SH | 21 | BX | DC | 1 | P | CCFtab longitudinal distance |
| 1 | 1 | DGVT | SH | 21 | BX | DC | 2 | P | CCFtab lateral distance |
| 1 | 2 | DGVT | SH | 20 | BX | DC | X | P | CCFtab headway distance |
| 1 | 2 | DCOL | MI | 00 | 00 | DC | 1 | P | longitudinal distance VUT middle to collision point |
| 2 | 1 | DCOL | BX | 00 | 00 | DC | 0 | P | absolute distance GVT corner to collision point |
| 1 | 2 | DVUT | SH | 20 | MI | DC | 1 | P | VUT-VRU longitudinal distance |
| 1 | 2 | DVUT | MI | 20 | MI | DC | 2 | P | VUT-VRU lateral distance |
| 1 | 2 | DVUT | MI | 20 | MI | DC | 0 | P | VUT-VRU absolut distance |
| 1 | 2 | DGVT | SH | 20 | SH | DC | 1 | P | relative longitudinal distance |
| 1 | 0 | DGVT | SH | 20 | SH | DC | 2 | P | relative lateral distance |
| 1 | 1 | DVUT | DR | 20 | SH | DC | 1 | P | lateral distance upper corner door to target shape |

## Codes for Test Type C2C

A list of examples gives the following table.

| Channel Code | Unit | RefSys | Description |
| --- | --- | --- | --- |
| 10PEAC000000000P | 1 | LOC | Accelerator Pedal Relative Position |
| 10PEAC00RW00000P | 1 | LOC | Accelerator Pedal Command |
| 10PEBR000000DS0P | m | LOC | Brake Pedal Position |
| 10PEBR000000FO0P | N | LOC | Brake Pedal Force |
| 10PEBR00RW00000P | 1 | LOC | Brake Pedal Command |
| 10STWL000000AV1P | rad/s | LOC | VUT Steering Wheel Angle Velocity |
| 10STWL000000AN1P | rad | LOC | VUT Steering Wheel Angle |
| 10STWL000000MO1P | Nm | LOC | VUT Steering Wheel torque |
| 10TFCW000000EV00 | 1 | - | FCW acoustical |
| 10TLDW000000EV00 | 1 | - | Curve entry event |
| 10TTTC000000TI00 | s | - | TTC |
| 10TTTC010000TI00 | s | - | TTC with Acceleration |
| 10TURN000000EV00 | 1 | - | Turning Indicator Event |
| 10VEHC000000ACXP | m/(s*s) | 1DY | VUT Longitudinal Acceleration |
| 10VEHC000000AVZP | rad/s | 1DY | VUT Yaw Rate |
| 10VEHC000000DSXP | m | TST | VUT Position X |
| 10VEHC000000DSYP | m | TST | VUT Position Y |
| 10VEHC000000VEXP | m/s | 1DY | VUT Longitudinal Velocity |
| 10VEHC00DI00DCYP | m | LOC | VUT Lateral Path Error |
| 10VEHC00DI00DSXP | m | 1DY | Headway Distance VUT - Target |
| 10VEHC00DI00DSYP | m | 1DY | Lateral Distance VUT - Target |
| 20VEHC000000ACXP | m/(s*s) | 2DY | Target Longitudinal Acceleration |
| 20VEHC000000AVZP | rad/s | 2DY | Target Yaw Rate |
| 20VEHC000000DSXP | m | TST | Target Position X |
| 20VEHC000000DSYP | m | TST | Target Position Y |
| 20VEHC000000VEXP | m/s | 2DY | Target Longitudinal Velocity |
| 20VEHC000000VEYP | m/s | 2DY | Target Lateral Velocity |
| 20VEHC00DI00DCYP | m | LOC | Target Lateral Path Error |
| … |  |  |  |
| Examples for the usage of bus signals: |  |  |  |
| 10TTTCRD0000TI00 | s | - | Bus Signal TTC |
| 10VEHCRD0000ACXP | m/(s*s) | 1DY | Bus Signal VUT Requested Acceleration |
| 102BUS010000000P | 1 | - | Bus Signal 1 Raw |
| 102BUS010000EV00 | 1 | - | Bus Signal 1 |
| 102BUS020000EV00 | 1 | - | Bus Signal 2 |
| 102BUS030000000P | 1 | - | Bus Signal 3 Raw |
| 102BUS030000EV00 | 1 | - | Bus Signal 3 |
| … |  |  |  |

## Codes for Test Type VRU

A list of examples gives the following table.

| Channel Code | Unit | RefSys | Description |
| --- | --- | --- | --- |
| 10PEAC000000000P | 1 | LOC | Accelerator Pedal Relative Position |
| 10PEAC00RW00000P | 1 | LOC | Accelerator Pedal Command |
| 10PEBR000000DS0P | m | LOC | Brake Pedal Position |
| 10PEBR000000FO0P | N | LOC | Brake Pedal Force |
| 10STWL000000AV1P | rad/s | LOC | VUT Steering Wheel Angle Velocity |
| 10STWL000000AN1P | rad | LOC | VUT Steering Wheel Angle |
| 10STWL000000MO1P | Nm | LOC | VUT Steering Wheel torque |
| 10TFCW000000EV00 | 1 | - | FCW acoustical |
| 10VEHC000000ACXP | m/(s*s) | 1DY | VUT Longitudinal Acceleration |
| 10VEHC000000AVZP | rad/s | 1DY | VUT Yaw Rate |
| 10VEHC000000DSXP | m | TST | VUT Position X |
| 10VEHC000000DSYP | m | TST | VUT Position Y |
| 10VEHC000000VEXP | m/s | 1DY | VUT Longitudinal Velocity |
| 10VEHC00DI00DCYP | m | LOC | VUT Lateral Path Error |
| 10VEHC00DI00DSXP | m | 1DY | Headway Distance VUT - Target |
| 10VEHC00DI00DSYP | m | 1DY | Lateral Distance VUT - Target |
| 20TWCA000000DSXP 20CYCL000000DSXP | m | TST | Cyclist Position X |
| 20TWCA000000DSYP 20CYCL000000DSYP | m | TST | Cyclist Position Y |
| 20TWCA000000VEXP 20CYCL000000VEXP | m/s | 2DY | Cyclist Longitudinal Velocity |
| 20TWCA000000VEYP 20CYCL000000VEYP | m/s | 2DY | Cyclist Lateral Velocity |
| 20TWCA00DI00DCYP 20CYCL00DI00DCYP | m | LOC | Cyclist Lateral Path Error |
| 20TWCA000000ANYP 20CYCL000000ANYP | m | LOC | Cyclist Yaw Angle |
| 20TWCA000000AVYP 20CYCL000000AVYP | m | LOC | Cyclist Yaw Velocity |
| 20PDAD000000DSXP 20PEDA000000DSXP | m | TST | Pedestrian (adult) Position X |
| 20PDAD000000DSYP 20PEDA000000DSYP | m | TST | Pedestrian (adult) Position Y |
| 20PDAD000000VEXP 20PEDA000000VEXP | m/s | 2DY | Pedestrian (adult) Longitudinal Velocity |
| 20PDAD000000VEYP 20PEDA000000VEYP | m/s | 2DY | Pedestrian (adult) Lateral Velocity |
| 20PDAD00DI00DCYP 20PEDA00DI00DCYP | m | LOC | Pedestrian (adult) Lateral Path Error |
| 20PDAD000000ANYP 20PEDA000000ANYP | m | LOC | Pedestrian (adult) Yaw Angle |
| 20PDAD000000AVYP 20PEDA000000AVYP | m | LOC | Pedestrian (adult) Yaw Velocity |
| 20PDC7000000DSXP 20PEDC000000DSXP | m | TST | Pedestrian (child) Position X |
| 20PDC7000000DSYP 20PEDC000000DSYP | m | TST | Pedestrian (child) Position Y |
| 20PDC7000000VEXP 20PEDC000000VEXP | m/s | 2DY | Pedestrian (child) Longitudinal Velocity |
| 20PDC7000000VEYP 20PEDC000000VEYP | m/s | 2DY | Pedestrian (child) Lateral Velocity |
| 20PDC700DI00DCYP 20PEDC00DI00DCYP | m | LOC | Pedestrian (child) Lateral Path Error |
| 20PDC7000000ANYP 20PEDC000000ANYP | m | LOC | Pedestrian (child) Yaw Angle |
| 20PDC7000000AVYP 20PEDC000000AVYP | m | LOC | Pedestrian (child) Yaw Velocity |
| 20TWMB000000DSXP | m | TST | Motorcycle Position X |
| 20TWMB000000DSYP | m | TST | Motorcycle Position Y |
| 20TWMB000000VEXP | m/s | 2DY | Motorcycle Longitudinal Velocity |
| 20TWMB000000VEYP | m/s | 2DY | Motorcycle Lateral Velocity |
| 20TWMB00DI00DCYP | m | LOC | Motorcycle Lateral Path Error |
| 20TWMB000000ANYP | m | LOC | Motorcycle Yaw Angle |
| 20TWMB000000AVYP | m | LOC | Motorcycle Yaw Velocity |
| … |  |  |  |
| Examples for the usage of bus signals: |  |  |  |
| 10TTTCRD0000TI00 | s | - | Bus Signal TTC |
| 10VEHCRD0000ACXP | m/(s*s) | 1DY | Bus Signal VUT Requested Acceleration |
| 102BUS010000000P | 1 | - | Bus Signal 1 Raw |
| 102BUS010000EV00 | 1 | - | Bus Signal 1 |
| 102BUS020000EV00 | 1 | - | Bus Signal 2 |
| 102BUS030000000P | 1 | - | Bus Signal 3 Raw |
| 102BUS030000EV00 | 1 | - | Bus Signal 3 |
| … |  |  |  |

## Codes for Test Type LSS

A list of examples gives the following table.

| Channel Code | Unit | RefSys | Description |
| --- | --- | --- | --- |
| 10TLCR000000TI00 | s | - | TLC |
| 10TLCRFRLE00EV00 | 1 | - | Line Crossing Time LHS |
| 10TLCRFRRI00EV00 | 1 | - | Line Crossing Time RHS |
| 10TLDW000000EV00 | 1 | - | LDW Activation Time |
| 10TLDW010000EV00 | 1 | - | LDW Vibration |
| 10TLKA000000EV00 | 1 | - | LKA Activation Time |
| 10VEHC000000ACXP | m/(s*s) | 1DY | VUT Longitudinal Acceleration |
| 10VEHC000000ACYP | m/(s*s) | 1DY | VUT Lateral Acceleration |
| 10VEHC000000AVZP | rad/s | 1DY | VUT Yaw Rate |
| 10VEHC000000DSXP | m | TST | VUT Position X |
| 10VEHC000000DSYP | m | TST | VUT Position Y |
| 10VEHC000000VEXP | m/s | 1DY | VUT Longitudinal Velocity |
| 10VEHC000000VEYP | m/s | 1DY | VUT Lateral Velocity |
| 10VEHC00DI00DCYP | m | LOC | Lateral Path Error VUT |
| 10VEHC00DI00DSYP | m | 1DY | Lateral Displacement |
| 11WHEL000000DSXP | m | TST | VUT Front Left Wheel Position X - needed? |
| 11WHEL000000DSYP | m | TST | VUT Front Left Wheel Position Y - needed? |
| 13WHEL000000DSXP | m | TST | VUT Front Right Wheel Position X - needed? |
| 13WHEL000000DSYP | m | TST | VUT Front Right Wheel Position Y - needed? |
| 20VEHC000000ACXP | m/(s*s) | 2DY | Target Longitudinal Acceleration |
| 20VEHC000000AVZP | rad/s | 2DY | Target Yaw Rate |
| 20VEHC000000DSXP | m | TST | Target Position X |
| 20VEHC000000DSYP | m | TST | Target Position Y |
| 20VEHC000000VEXP | m/s | 2DY | Target Longitudinal Velocity |
| … |  |  |  |
| Examples for the usage of bus signals: |  |  |  |
| 10TLKARD0000000P | 1 | - | Bus Signal LKA Requested |
| 10TLKARD0000EV00 | 1 | - | Bus Signal LKA Approval |
| 10VEHCRDDI00DSYP | m | 1DY | Bus Signal Lateral Displacement |
| … |  |  |  |

## Codes for Test Type DOW

A list of examples gives the following table.

| Channel Code | Unit | RefSys | Description |
| --- | --- | --- | --- |
| 1?TDOP000000EV00 | 1 | - | Door opening interface SP X (Contact sensor) |
| 1?TDOPWA0000EV00 | 1 | - | Door opening interface signal SP X |
| 1?TDOPWA0200EV00 | 1 | - | Door opening warning acoustical SP X |
| 1?TDOPWA0300EV00 | 1 | - | Door opening warning optical SP X |

# Examples

## Examples for MME 1.6

### Example of a test information file MME 1.6

Filename: 26C2C-123M.mme

Data format edition number  :1.6

Laboratory name             :NOVALUE

Laboratory contact name     :NOVALUE

Laboratory contact phone    :NOVALUE

Laboratory contact fax      :NOVALUE

Laboratory contact email    :NOVALUE

Laboratory test ref. number :26C2C-123M

Customer name               :NOVALUE

Customer test ref. number   :NOVALUE

Customer project ref. number:NOVALUE

Customer order number       :NOVALUE

Customer cost unit          :NOVALUE

Customer test engineer name :NOVALUE

Customer test engineer phone:NOVALUE

Customer test engineer fax  :NOVALUE

Customer test engineer email:NOVALUE

Title                       :NOVALUE

Medium No./number of media  :1/1

Timestamp                   :2026-07-11 08:41:35

Type of the test            :C2C

Subtype of the test         :CCRb_AEB

Regulation                  :EuroNCAP_C2C_2020

Scenario                    :CCRb

System Function             :AEB

Test Campaign Type          :C2C

Test Campaign Name          :21C2C

Environment day light       :day

Precipitation type          :NOVALUE

Precipitation intensity     :NOVALUE

Weather obstruction         :NOVALUE

Weight of obstruction       :NOVALUE

Wind speed                  :NOVALUE

Road surface temperature    :NOVALUE

Sky conditions              :NOVALUE

Road conditions             :NOVALUE

Test completion             :NOVALUE

Position of the sun         :NOVALUE

Glare angle                 :NOVALUE

Reference temperature       :NOVALUE

Relative air humidity       :NOVALUE

Date of the test            :11-07-2026 08:40:30

Type of data source         :Physical Test

Region                      :EU

Robustness Layer            :NOVALU

Run repetition              :1

Number of test objects      :2

Name of test object 1       :VUT

Velocity test object 1      :11.2

Mass test object 1          :NOVALUE

Driver position object 1    :NOVALUE

Impact side test object 1   :FR

Test function side 1        :FR

Type of test object 1       :1

Class of test object 1      :VUT

Code of test object 1       :NOVALUE

Ref. number of test object 1:NOVALUE

Velocity intended TOB 1     :11.1111

Velocity intended lat. TOB 1:0

Impact Location TOB 1       :50

Velocity lateral TOB 1      :0

Acceleration int. TOB 1     :NOVALUE

Acceleration int. lat. TOB 1:NOVALUE

Lane Departure Side TOB 1   :NOVALUE

Headway distance int. TOB 1 :12

Dist. Offset ini. Pos. TOB 1:NOVALUE

Time Offset ini. Pos. TOB 1 :NOVALUE

Intended Heading TOB 1      :0

Lane marking function line  :NOVALUE

Lane marking secondary line :NOVALUE

Vel. int. lane dep. TOB 1   :NOVALUE

Distance eye to end TOB 1   :NOVALUE

Shape Front TOB 1           :(-0.9;-0.2) (-0.6;-0.1) (-0.3;-0.05) (0;0) (0.3;-0.005) (0.6;-0.1) (0.9;-0.2)

Shape Rear TOB 1            :(0.9;-3.910) (0.9;3.300) (0.9;2.800) (0.9;1.960) (0.9;1.340) (0.9;0.720) (0.9;-0.2)

Shape Right Side TOB 1      :(-0.9;-3.910) (-0.9;3.300) (-0.92.800y3) (-0.9;1.960) (-0.9;1.340) (-0.9;0.720) (-0.9;-0.2)

Shape Left Side TOB 1       :(-0.9;-3.910) (-0.6;-4.010) (-0.3;-4.110) (0;-4.210) (0.3;-4.110) (0.6;-4.010) (0.9;-3.910)

Dimensions TOB 1            :(4.210;1.820)

Front overhang TOB 1        :750

Driver State TOB 1          :NOVALUE

S/W version TOB 1           :V15

Name of test object 2       :GVT

Velocity test object 2      :0

Mass test object 2          :NOVALUE

Driver position object 2    :NOVALUE

Impact side test object 2   :RE

Type of test object 2       :2

Class of test object 2      :GVT

Code of test object 2       :NOVALUE

Ref. number of test object 2:NOVALUE

Velocity intended TOB 2     :11.1111

Velocity intended lat. TOB 2:0

Impact Location TOB 2       :50

Test function side 2        :

Velocity lateral TOB 2      :

Acceleration int. TOB 2     :NOVALUE

Acceleration int. lat. TOB 2:NOVALUE

Lane Departure Side TOB 2   :NOVALUE

Headway distance int. TOB 2 :NOVALUE

Dist. Offset ini. Pos. TOB 2:NOVALUE

Time Offset ini. Pos. TOB 2 :NOVALUE

Intended Heading TOB 2      :0

Lane marking function line  :NOVALUE

Lane marking secondary line :NOVALUE

Vel. int. lane dep. TOB 2   :NOVALUE

Distance eye to end TOB 2   :NOVALUE

Shape Front TOB 2           :NOVALUE

Shape Rear TOB 2            :NOVALUE

Shape Right Side TOB 2      :NOVALUE

Shape Left Side TOB 2       :NOVALUE

Dimensions TOB 2            :NOVALUE

Front overhang TOB 2        :NOVALUE

Driver State TOB 2          :NOVALUE

S/W version TOB 2           :NOVALUE

### Example of a channel information file MME 1.6

Filename: 21C2C-123M.chn

Instrumentation standard    :NOVALUE

Number of channels          :16

Name of channel 001         :10VEHC000000ACXP / VUT Longitudinal Acceleration

Name of channel 002         :10VEHC000000VEXP / VUT Longitudinal Velocity

Name of channel 003         :10VEHC000000VEYP / VUT Lateral Velocity

Name of channel 004         :10VEHC000000AVZP / VUT Yaw Rate

Name of channel 005         :10VEHC00DI00DCYP / Lateral path error VUT

Name of channel 006         :10STWL000000MO1P / VUT Steering Wheel Torque

Name of channel 007         :10TLDW000000EV00 / LDW Activation Time

Name of channel 008         :10TLKA000000EV00 / LKA Activation Time

Name of channel 009         :10VEHC00DI00DSYP / Distance Line - VUT Lateral

Name of channel 010         :11WHEL000000DSXP / VUT Front Left Wheel Position X

Name of channel 011         :11WHEL000000DSYP / VUT Front Left Wheel Position Y

Name of channel 012         :13WHEL000000DSXP / VUT Front Right Wheel Position X

Name of channel 013         :13WHEL000000DSYP / VUT Front Right Wheel Position Y

Name of channel 014         :20VEHC000000ACXP / GVT Longitudinal Acceleration

Name of channel 015         :20VEHC000000VEXP / GVT Longitudinal Velocity

Name of channel 016         :20VEHC000000AVZP / GVT Yaw Rate

### Example of a channel data file MME 1.6

Filename: 21C2C-123M.008

Test object number          :1

Data source                 :transducer

Data status                 :ok

Name of the channel         :X-Position of the VUT

Laboratory channel code     :10VEHC000000DSXP

Customer channel code       :VUT Position X

Reference channel           :implicit

Reference channel name      :NOVALUE

Channel code                :10VEHC000000DSXP

Channel frequency class     :NOVALUE

Unit                        :m

Reference system            :TST

Transducer type             :NOVALUE

Pre-filter type             :NOVALUE

Cut off frequency           :NOVALUE

Channel amplitude class     :NOVALUE

Sampling interval           :0.01

Bit resolution              :NOVALUE

Time of first sample        :0.01

Number of samples           :2406

Start offset interval       :NOVALUE

End offset interval         :NOVALUE

Offset post test            :NOVALUE

Transducer id               :NOVALUE

Transducer natural frequency:NOVALUE

Transducer damping ratio    :NOVALUE

0.01

0.02

0.03

…

## Examples for MME 2.1

### Example of a test information file MME 2.1

Filename: 21C2C-123M.mme

Data format edition number  :2.1

Timestamp                   :2021-04-14T06:41:35+02:00

Laboratory name             :NOVALUE

Laboratory contact name     :NOVALUE

Laboratory contact phone    :NOVALUE

Laboratory contact fax      :NOVALUE

Laboratory contact email    :NOVALUE

Laboratory test ref number  :21C2C-123M_CARID_20210414_145239

Customer name               :NOVALUE

Customer test ref number    :NOVALUE

Customer project ref number :NOVALUE

Customer order number       :NOVALUE

Customer cost unit          :NOVALUE

Customer contact name       :NOVALUE

Customer contact phone      :NOVALUE

Customer contact fax        :NOVALUE

Customer contact email      :NOVALUE

Title                       :NOVALUE

Type of the test            :C2C

Subtype of the test         :CCRs_AEB

Regulation                  :EuroNCAP_C2C_2020

Reference temperature       :NOVALUE

Relative air humidity       :NOVALUE

Date of the test            :12-04-2021

Number of test objects      :2

#Begin of testobjects

Type                        :1

Filename                    :21C2C-123M_1.mmi

#End of testobject

#Begin of testobject

Type                        :2

Filename                    :21C2C-123M_2.mmi

#End of testobject

### Examples of object information files MME 2.1

Filename: 21C2C-123M_1.mmi

Name                        :VUT

Velocity                    :4.17

Mass                        :NOVALUE

Driver position             :NOVALUE

Impact side test            :FR

Class                       :SV

Code                        :NOVALUE

Ref number                  :NOVALUE

Offset                      :NOVALUE

Filename: 21C2C-123M_2.mmi

Name                        :GVT

Velocity                    :0

Mass                        :NOVALUE

Driver position             :NOVALUE

Impact side test            :NOVALUE

Class                       :VT

Code                        :NOVALUE

Ref number                  :NOVALUE

Offset                      :NOVALUE

### Example of a channel information file MME 2.1

Filename: 21C2C-123M_Channel.mmi

Number of channels          :16

Data origin                 :T

#Begin of channel

Extended channel code       :10VEHC000000ACXP_1DYT

Channel code                :10VEHC000000ACXP

Reference system id         :1DY

#End of channel

#Begin of channel

Extended channel code       :10VEHC00DI00DCYP_LOCT

Channel code                :10VEHC00DI00DCYP

Reference system id         :LOC

#End of channel

#Begin of channel

Extended channel code       :10VEHC00DI00DSXP_1DYT

Channel code                :10VEHC000000DSXP

Reference system id         :1DY

#End of channel

#Begin of channel

Extended channel code       :10VEHC000000DSXP_TSTT

Channel code                :10VEHC000000DSXP

Reference system id         :TST

#End of channel

#Begin of channel

Extended channel code       :10VEHC000000DSYP_TSTT

Channel code                :10VEHC000000DSYP

Reference system id         :TST

#End of channel

…

### Example of a channel data file MME 2.1

Filename: 21C2C-123M_10VEHC000000DSXP_TSTT.mmd

Data structure              :Channel

Instrumentation standard    :NOVALUE

Data source                 :transducer

Data status                 :ok

Unit                        :m

Name of the channel         :10VEHC000000DSXP

Laboratory channel code     :10VEHC000000DSXP

Customer channel code       :VUT Position X

Reference channel           :implicit

Reference channel name      :NOVALUE

Uuid                        :NOVALUE

Transducer type             :NOVALUE

Transducer id               :NOVALUE

Transducer natural frequency:NOVALUE

Transducer damping ratio    :NOVALUE

Calibration date            :NOVALUE

Calibration due date        :NOVALUE

Bit resolution              :NOVALUE

Cut off frequency           :NOVALUE

Channel frequency class     :Prefiltered

Channel amplitude class     :NOVALUE

Prefilter type              :NOVALUE

Sampling interval           :0.01

Time of first sample        :0.01

Number of samples           :2406

Start offset interval       :NOVALUE

End offset interval         :NOVALUE

Offset post test            :NOVALUE

#Start of data

0.01

0.02

0.03

…